# Assessment Requirements for MEM19047 Set gems in claw and bezel style settings

Release: 1

Assessment Requirements for MEM19047 Set gems in claw and bezel style settings

# Modification History

Release 1. Supersedes and is equivalent to MEM19047A Set gems in claw and bezel style settings.

# Performance Evidence

Evidence required to demonstrate competence in this unit must be relevant to and satisfy the requirements of the elements and performance criteria and include:

following work instructions, standard operating procedures (SOPs) and safe work practices

identifying metals and gems

applying techniques for handling gems safely and minimising stress during setting processes

producing claw and bezel settings according to industry standards on at least two occasions using setting tools and equipment

calculating required material size

securing the jewellery item to be set without inflicting damage

selecting the appropriate height and depth at which to cut the bearing on the wire of the claw setting on at least two occasions

seating the gem level and straight in the claw setting

securing the gem by pushing the claw wire over the gem

finishing the top of the claw to the desired shape on at least two occasions

selecting the appropriate height and depth at which to cut the bearing on the inside of the bezel wall on at least two occasions

seating the gem level and straight in the bezel setting

securing the gem by pushing the bezel wall over the gem

finishing the top of the bezel to the desired shape on at least two occasions

completing workplace documentation in accordance with procedures.

Note: Where a volume and/or frequency is not specified, demonstration must be provided at least once.

# Knowledge Evidence

Evidence required to demonstrate the required knowledge for this unit must be relevant to and satisfy the requirements of the elements and performance criteria and include knowledge of:

safe work practices and procedures and use of personal protective equipment (PPE)

setting types and terminology

bezel characteristics, including full and part bezels, along with bezel construction types

advantages and disadvantages of 2,4,6,8, and 12 wire settings and ‘V’ wire claw settings

gem materials, features and handling procedures

setting tools and equipment

maintenance requirements of tools

setting techniques, processes and procedures

working properties of materials

types of gems and their key characteristics

range of facet cuts used on gems

qualities, features, limitations and working characteristics of different metals used in jewellery

suitability of metal for carving and setting

essential characteristics of wire claw settings

essential characteristics of different styles of bezel settings

various securing techniques for jewellery items to be set.

# Assessment Conditions

Assessors must:

have vocational competency in setting gems in claw and bezel style settings at least to the level being assessed with relevant industry knowledge and experience

satisfy the assessor requirements in the Standards for Registered Training Organisations 2015 or its replacement and comply with the National Vocational Education and Training Regulator Act 2011, its replacement or equivalent legislation covering VET regulation in a non-referring state/territory as the case requires.

Where possible, assessment must occur in operational workplace situations. Where this is not possible or where personal safety or environmental damage are limiting factors, assessment must occur in a sufficiently rigorous simulated environment that reflects realistic operational workplace conditions. This must cover all aspects of workplace performance, including environment, task skills, task management skills, contingency management skills and job role environment skills.

Conditions for assessment must include access to all tools, equipment, materials and documentation required, including relevant workplace procedures, product and manufacturing specifications.

Assessment processes and techniques must be appropriate to the language, literacy and numeracy requirements of the work being performed and the needs of the candidate.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b7050d37-5fd0-4740-8f7d-3b7a49c10bb2