# MEM19064 Cut gemstone spheres and beads

Release: 1

MEM19064 Cut gemstone spheres and beads

# Modification History

Release 1. New unit.

# Application

This unit of competency defines the skills and knowledge required to produce gemstone spheres and beads, including the selection of the rough gemstone through to the drilling and final polishing. It is based on deleted unit CUVOPA404A Cut opal spheres and beads.

This unit applies to individuals processing gemstones in a small or large-scale production context. They use and adapt technical knowledge and skills to produce work.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

### Band: B

### Unit Weight: 3

# Pre-requisite Unit

MEM13015 Work safely and effectively in manufacturing and engineering

MEM16006 Organise and communicate information

# Competency Field

Jewellery

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Determine job requirements | 1.1 Follow standard operating procedures (SOPs) 1.2 Comply with work health and safety (WHS) requirements at all times 1.3 Identify job requirements from specifications 1.4 Investigate methods of making spheres and beads |
| 2. Prepare to use bead and sphere making equipment | 2.1 Set up bead mill, sphere-making equipment and drills in accordance with manufacturer’s specifications 2.2 Complete manufacturer’s maintenance checklist 2.3 Select materials suitable for making spheres or beads 2.4 Pre-form gemstones using saw or grinder to produce required shape 2.5 Grind shape corners to establish rounded form dop stone and work on pavilion first |
| 3. Produce and drill gemstones beads | 3.1 Use appropriate holder templates throughout the process 3.2 Grind pre-formed beads in bead mill until they are rounded 3.3 Remove and clean bead materials discarding faulty or broken gemstones 3.4 Select required beads for drilling 3.5 Place bead in bead vice, drill half way into the bead, turn over, check alignment and drill the other half ensuring a cut hole through the bead |
| 4. Polish beads | 4.1 Grind beads in a tumble polisher until requirements are met, using increasingly fine grinding medium 4.2 Clean tumbler and beads at every change over 4.3 Clean beads thoroughly after the final grind 4.4 Place ground beads in the tumbler with polishing medium and tumble to a high lustre 4.5 Clean finished beads thoroughly with water to meet requirements |
| 5. Produce gemstone spheres | 5.1 Select gemstones to make spheres and pre-form the spheres 5.2 Grind the spheres in a sphere maker with water and increasingly fine grinding medium 5.3 Clean spheres thoroughly at each change over 5.4 Polish cleaned sphere with polishing medium and water using leather sphere cups |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential to performance.

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Range of Conditions

| This field allows for different work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) are included. |  |
| --- | --- |
| Specifications include one or more of the following: | client requirements job sheets work instructions. |
| Maintenance areas include: | bead mill: all bearings all electrics bead holders and wear factor bottom rotating disc and wear factor rubber pad on top rotating disc sphere makers: all electrics assorted sphere cups polishing pads reduction boxes. |
| Materials suitable for making spheres and beads include one or more of the following: | any gemstone material, taking into account a yield factor or recovery rate of only 10% to 15% from a given ounce of rough gemstone beads of a variety of regular or free form shapes spheres cut from any gemstone material, with consideration of yield factor. |
| Grinding mediums include one or more of the following: | diamond powder (#80 through to #1200) silicon carbide grit (#80 through to #1200). |
| Polishing mediums include one or more of the following: | cerium or tin oxide diamond powder (#3000 through to #100,000). |

# Unit Mapping Information

No equivalent unit.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b7050d37-5fd0-4740-8f7d-3b7a49c10bb2