# MEM20012 Service and repair mechanical automotive locking systems

Release: 1

MEM20012 Service and repair mechanical automotive locking systems

# Modification History

Release 1. Supersedes and is equivalent to MEM20012A Service and repair mechanical automotive locking systems

# Application

This unit of competency defines the skills and knowledge required to identify service and repair requirements for mechanical automotive locking systems on commercial and private use vehicles.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

### Band:

This unit has dual status and is to be regarded as both a Specialisation Band A unit and Specialisation Band B unit for progression to C5 (AQF level V).

### Unit Weight:	4

# Pre-requisite Unit

| MEM11011 | Undertake manual handling |
| --- | --- |
| MEM13015 | Work safely and effectively in manufacturing and engineering |
| MEM16006 | Organise and communicate information |
| MEM18001 | Use hand tools |
| MEM18002 | Use power tools/hand held operations |
| MEM20001 | Produce keys |
| MEM20002 | Assemble and test lock mechanisms |
| MEM20006 | Maintain and service mechanical locking devices |

# Competency Field

Locksmithing

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe the performance needed to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Determine job requirements | 1.1 | Follow standard operating procedures (SOPs) |
|  |  | 1.2 | Comply with work health and safety (WHS) requirements at all times |
|  |  | 1.3 | Select and use appropriate personal protective equipment (PPE) in accordance with SOPs |
|  |  | 1.4 | Identify job requirements from specifications, job sheets or work instructions |
| 2 | Identify service and repair requirements | 2.1 | Establish ownership and verify in accordance with organisational and industry procedures |
|  |  | 2.2 | Obtain system and component specifications and other relevant information from appropriate sources and in accordance with organisational and customer requirements |
|  |  | 2.3 | Establish nature of damage in accordance with organisational procedures, established inspection/diagnostic techniques and original specifications |
|  |  | 2.4 | Establish service and repair requirements and other information relevant to task in accordance with customer needs, organisational and manufacturer specifications and legislation, codes, regulations and standards |
| 3 | Repair, replace and service automotive locking system | 3.1 | Identify risks and hazards and make area secure |
|  |  | 3.2 | Apply specific isolation and safety measures with special regard to associated equipment |
|  |  | 3.3 | Select tools, equipment, materials and consumables, check for serviceability and use |
|  |  | 3.4 | Inspect lock assemblies and components and remove, repair, service and replace as appropriate and practicable, in accordance with SOPs, industry practices and factors impacting on feasibility of repair |
|  |  | 3.5 | Inspect sub-assemblies and assemblies, test and adjust as necessary for compliance to original factory assembly and functioning |
| 4 | Finalise servicing process | 4.1 | Check function of entire locking system for compliance with operational specifications and adjust, as necessary |
|  |  | 4.2 | Complete documentation and process in accordance with legislative, assignment and organisational requirements |
|  |  | 4.3 | Clean work area, tools and equipment and store in accordance with SOPs |
|  |  | 4.4 | Clean vehicle and return to serviceable condition according to customer and organisational requirements |

# Foundation Skills

This section describes those required skills (reading, writing, oral communication and numeracy) that are essential to workplace performance in this unit of competency.

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Range of Conditions

| This field allows for different work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) are included. |  |
| --- | --- |
| PPE includes one (1) or more of the following: | face shields, goggles and safety glasses hearing protection dust masks and respirators high visibility vests or clothing head protection sun protection, hats and sun screen gloves and knee pads safety footwear witches hats warning signs and warning tapes temporary safety barriers fire extinguisher first aid kit |
| Appropriate sources of information include one (1) or more of the following: | catalogues manufacturer manuals drawings detailed/technical sketches computer software/internet colleagues/industry experts associated data sheets |
| Other information includes one (1) or more of the following: | schedules timeframes access and site information products installation and upgrade requirements specific client requests materials costings warranties and service information legislation, codes and regulatory requirements WHS requirements |
| Customer requirements include one (1) or more of the following: | function and capabilities access issues service and maintenance requirements product operation and warranty information costings |
| Risks and hazards include one (1) or more of the following: | accidental triggering of supplemental restraint systems (SRS) (air bags) unique dangers arising from working on the roadside non-compliance with manufacturer recommendations, materials handling, chemical and environmental physical hazards |
| Associated equipment includes one (1) or more of the following: | air bags and other SRS equipment steering locking devices anti-theft devices |
| Lock assemblies and components include one (1) or more of the following: | in-line pin single and double sided wafer 2 and 4 track tibbe (rotating disc) dimple sidebar split wafer new and other mechanisms |
| Tools and equipment include one (1) or more of the following: | hand tools power tools and accessories security driver bits fixing tools special-purpose equipment trim removal tools fibre optic scope soldering iron vacuum cleaner |
| Materials and consumables include one (1) or more of the following: | lubricants adhesives and sealants cleaning materials wire and cable insulation tape/shrink tube/electrical connectors solder electronic components |
| Documentation includes one (1) or more of the following: | historical records procedures and measurements for opening specific vehicles job sheets invoices warranties |

# Unit Mapping Information

Release 1. Supersedes and is equivalent to MEM20012A Service and repair mechanical automotive locking systems

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b7050d37-5fd0-4740-8f7d-3b7a49c10bb2