# MEM21021 Restore clockwork mechanisms

Release: 1

MEM21021 Restore clockwork mechanisms

# Modification History

Release 1. Supersedes and is equivalent to MEM21021A Restore clockwork mechanisms

# Application

This unit of competency defines the skills and knowledge required to restore and repair clock components by manufacturing parts using hand skills and machine tools and also covers servicing power equalisation systems.

Application includes restoration and repair work to antique clocks where parts are no longer available.

Where machine manufacture of clock components is required unit MEM21022 Manufacture watch and clock components should also be selected.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

#### Band:  B

#### Unit weight:  6

# Pre-requisite Unit

| MEM06007 | Perform basic incidental heat/quenching, tempering and annealing |
| --- | --- |
| MEM09002 | Interpret technical drawing |
| MEM11011 | Undertake manual handling |
| MEM13015 | Work safely and effectively in manufacturing and engineering |
| MEM16006 | Organise and communicate information |
| MEM18001 | Use hand tools |
| MEM21017 | Service and repair clock timepieces |
| MEM21019 | Service and repair clock striking mechanisms |
| MEM21020 | Service and repair clock chiming mechanisms |

# Competency Field

Horology

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe the performance needed to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Determine job requirements | 1.1 | Follow standard operating procedures (SOPs) |
|  |  | 1.2 | Comply with work health and safety (WHS) requirements at all times |
|  |  | 1.3 | Select and use appropriate personal protective equipment (PPE) in accordance with SOPs |
|  |  | 1.4 | Identify job requirements from specifications, drawings, job sheets or work instructions |
| 2 | Identify clock restoration requirements | 2.1 | Identify missing, damaged or worn parts |
|  |  | 2.2 | Identify prior modifications and repairs, including non-standard or non-original components |
|  |  | 2.3 | Plan repairs in consideration of original clock design and manufacture |
|  |  | 2.4 | Record and document repair |
| 3 | Service and repair power equalisation devices | 3.1 | Identify fusee and stopwork device |
|  |  | 3.2 | Release mainspring power |
|  |  | 3.3 | Diagnose system faults |
|  |  | 3.4 | Replace fusee chains or cables |
|  |  | 3.5 | Set up fusee and stopwork systems |
| 4 | Produce clock parts by hand | 4.1 | Prepare working drawings and templates of replacement parts |
|  |  | 4.2 | Plan suitable method and sequence of production |
|  |  | 4.3 | Select materials suitable for manufacturing |
|  |  | 4.4 | Transfer designs and mark out templates and work pieces |
|  |  | 4.5 | Select appropriate hand tools to undertake process |
|  |  | 4.6 | Produce components by hand to required specifications |
|  |  | 4.7 | Conduct finishing, heat treatment and polishing of components |
|  |  | 4.8 | Verify function of manufactured components |
| 5 | Refinish and polish clock components | 5.1 | Select appropriate refinishing process to repair worn or corroded clock components |
|  |  | 5.2 | Refinish parts to replicate original finish |
|  |  | 5.3 | Adjust refinished components for correct function |
| 6 | Repair gears, wheels and arbors | 6.1 | Identify worn or damaged teeth |
|  |  | 6.2 | Fit, file and shape replacement teeth |
|  |  | 6.3 | Re-pivot arbors |
|  |  | 6.4 | Secure replacement part using appropriate fixing method |
|  |  | 6.5 | Test-run with matched component |

# Foundation Skills

This section describes those required skills (reading, writing, oral communication and numeracy) that are essential to workplace performance in this unit of competency.

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Range of Conditions

| This field allows for different work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) are included. |  |
| --- | --- |
| Clock design and manufacture includes one (1) or more of the following: | remaining faithful to original design and finish accounting for value and provenance maintaining originality of parts |
| Record and document repair includes one (1) or more of the following: | date and extent of repair cost of replacement part time spent on procedure |
| Working drawings and templates include one (1) or more of the following: | sketch or illustration by hand photocopy original component then adhere to stock |
| Appropriate tools and equipment include one (1) or more of the following: | files burnishers oil stones jewellers saw finishing papers needle files marking out equipment lathe/drilling tailstock |
| Produce components by hand includes the use of one (1) or more of the following: | pallets striking and chiming levers ratchet wheels clickwork taper pins |
| Heat treatment includes one (1) or more of the following: | annealing hardening tempering (bluing) |
| Refinishing process includes one (1) or more of the following: | filing honing graining polishing |
| Clock components include one (1) or more of the following: | arbors and pinions clicks pallets screws taper pins |
| Fixing method includes one (1) or more of the following: | Loctite low melting solder electronic soldering iron spirit lamp |

# Unit Mapping Information

Release 1. Supersedes and is equivalent to MEM21021A Restore clockwork mechanisms

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b7050d37-5fd0-4740-8f7d-3b7a49c10bb2