# MEM24014 Perform thermography tests

Release: 1

MEM24014 Perform thermography tests

# Modification History

Release 1. New unit.

# Application

This unit of competency has been developed for training of non-destructive testing (NDT) technicians in performing thermography (infra-red) non-destructive tests, including quantitative and qualitative thermography testing.

The unit defines the skill and knowledge for passive and active testing of materials, the processing of infra-red images, interpretation of results, report preparation and storage for later comparative analysis.

Where interpretation of technical drawings is required, unit MEM09002 Interpret technical drawing should also be selected.

Where the selection and use of engineering measurement is required, unit MEM12023 Perform engineering measurements should also be selected.

Competence in this unit along with MEM30044 Apply materials technology principles to non-destructive testing may satisfy the requirements of AS 18436.7: Condition monitoring and diagnostics of machines - Requirements for qualification and assessment of personnel – Part 7: Thermography.

No licensing, legislative or certification requirements apply to this unit at the time of publication. Certification against Australian standards may be achieved where assessment in this unit of competency is carried out in conjunction with an examining authority as described in ISO 9712 Non-destructive testing — Qualification and certification of personnel.

#### Band: B

#### Unit Weight: 4

# Pre-requisite Unit

MEM24020 Apply materials technology principles to non-destructive testing

MEM30007 Select common engineering materials

# Competency Field

Non-destructive testing

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Determine job requirements | 1.1 Follow standard operating procedures (SOPs) 1.2 Comply with work health and safety (WHS) requirements at all times 1.3 Use appropriate personal protective equipment (PPE) in accordance with SOPs 1.4 Identify job requirements from customer specifications, job sheets or work instructions |
| 2. Prepare for thermographic testing | 2.1 Liaise with client to ensure safe and effective access to test area and test material 2.2 Identify test material inspection area 2.3 Review relevant standards and codes for requirements relevant to thermographic testing task 2.4 Clean and prepare material for testing using appropriate procedures 2.5 Check and prepare all test equipment in accordance with manufacturers specifications and job specification. |
| 3. Perform thermographic testing | 3.1 Select the most appropriate magnetic particle test for the material and/or application 3.2. Set up test equipment in accordance with relevant standards and job specifications 3.3 Carry out thermographic test in accordance with relevant standards and specifications 3.4 Check thermographic testing equipment for defects, and maintain and store in accordance with procedures and manufacturer instructions |
| 4. Interpret and report the results of thermographic tests | 4.1 Process thermographic images in accordance with standards and codes and job specifications 4.2 Determine material condition and defects according to standards and codes and job specification acceptance criteria 4.3 Report test results in accordance with enterprise procedures, accepted industry practices and job specifications 4.4 Store and catalogue thermal images for later comparative analysis |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential to performance.

Other foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Range of Conditions

| This field allows for different work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) are included. |  |
| --- | --- |
| Standards and codes include Australian or international Standards and codes and include one or more of the following: | non-destructive testing of welds non-destructive testing of materials standards for materials used in manufacturing and engineering standards for calibration and operation of test equipment. |
| Test material include one or more of the following: | ferrous and nonferrous metal: castings plate pipe welds tanks manufactured components other ferrous and nonferrous items glass cement and concrete polymers and elastomers composites textiles. |
| Environment conditions include: | direct sunlight precipitation fog, mist and ice wind and draught. |

# Unit Mapping Information

No equivalent unit

# Links

Companion Volume Implementation Guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b7050d37-5fd0-4740-8f7d-3b7a49c10bb2

Assessment Requirements for MEM24014 Perform thermography tests

# Modification History

Release 1. New unit.

# Performance Evidence

There must be evidence the candidate has demonstrated the ability to:

follow work instructions, standard operating procedures (SOPs) and safe work practices

identify and interpret specifications, relevant testing standards, codes and other reference documentation for performing thermographic testing

prepare work areas and materials for thermographic testing

adjust thermographic testing settings and techniques for environment conditions and ambient temperature

select appropriate thermography techniques for at least three different materials and test specifications

undertake numerical operations and calculations associated with performing thermographic testing

perform the following thermographic tests on different materials and prepare a report for each test according to non-destructive standard and code requirements:

lock-in thermographic test (periodic excitation method)

pulse thermographic test

transient thermographic test

continual excitation thermographic test.

Where a volume and/or frequency is not specified, demonstration must be provided at least once.

# Knowledge Evidence

There must be evidence the candidate has knowledge of:

standards and codes relevant to non-destructive testing

applications and limitations of thermographic non-destructive testing for:

detection of defects in materials including cracks, cavities, voids, and inhomogeneities

inspection of welded joints

determination of material internal structure

detection of contaminants

condition monitoring

electromagnetic spectrum and wavelength ranges for infra-red, visible light, ultra-violet, X-ray and gamma ray

principles of heat and heat transfer including:

heat as energy

temperature scales

phases

heat transfer – conduction, convection and radiation including:

Fourier’s law

Newton’s law of cooling

evaporation fundamentals

principles of infra-red radiation including:

natural and artificial sources of infra-red radiation

principles related to emission, reflection, transmission and absorption of infra-red radiation

black body, grey body and selective radiators

Plank’s law

Wien’s law

Stefan-Boltzmann law

Kirchoff’s radiation law

cavity radiation effect

thermal property of materials including:

emissivity

reflectance

transmittance

effects of surface structure and coating

methods of temperature measurement

qualitative thermography including:

definition

advantages and disadvantages

typical applications in manufacturing and engineering

equipment and test techniques

quantitative thermography including:

definition

effects of temperature and the environment

advantages and disadvantages

typical applications in manufacturing and engineering

equipment and test techniques

difference between passive and active thermography

principles of excitation including:

sources

type of excitation – pulse, periodic or continuous

thermographic image capture including:

infra-red cameras including high-speed cooled infra-red cameras, lenses and other accessories

bolometers and other sensors

equipment factors affecting thermographic image capture including:

sensitivity

imager field of view

imager smallest identifiable object

imager smallest measurable object

emissivity of test material

focus

duration

temperature range setting

environment and temperature of test area

speed of response

frame repetition rate

thermographic image analysis techniques including:

lock-in

motion compensation

background and other subtractions

data selection criteria according to job type and standards, codes and procedures

use of image analysis software

thermographic image post-processing techniques.

# Assessment Conditions

Assessors must:

have vocational competency in performing thermographic testing at least to the level being assessed with relevant industry knowledge and experience

satisfy the assessor requirements in the Standards for Registered Training Organisations 2015 or its replacement and comply with the National Vocational Education and Training Regulator Act 2011, its replacement or equivalent legislation covering VET regulation in a non-referring state/territory as the case requires.

Where possible, assessment must occur in operational workplace situations. Where this is not possible or where personal safety or environmental damage are limiting factors, assessment must occur in a sufficiently rigorous simulated environment that reflects realistic operational workplace conditions that cover all aspects of workplace performance, including environment, task skills, task management skills, contingency management skills and job role environment skills.

There must be access to all tools, equipment, materials and documentation required, including relevant workplace procedures, product and manufacturing specifications.

Assessment processes and techniques must be appropriate to the language, literacy and numeracy requirements of the work being performed and the needs of the candidate.

# Links

Companion Volume Implementation Guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b7050d37-5fd0-4740-8f7d-3b7a49c10bb2