# MEM25005 Construct and assemble marine vessel timber components

Release: 2

MEM25005 Construct and assemble marine vessel timber components

# Modification History

Release 2. Quantum of hours of workplace practice removed. Supersedes and is equivalent to MEM25005 Construct and assemble marine vessel timber components (Release 1).

Release 1. Supersedes and is equivalent to MEM25005B Construct and assemble marine vessel timber components.

# Application

This unit of competency defines the skills and knowledge required to construct and assemble marine timber components, including laminating by cold processes, and applies to marine vessel backbone components.

Where hull is to be planked, planking can include carvel, strip, clinker and double diagonal systems.

Where permanent timber bending practices are required including boiling/steam bending then see unit MEM25009 Form timber shapes using hot processes.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

Band:	A

Unit Weight:	8

# Pre-requisite Unit

| MEM04018 | Perform general woodworking machine operations |
| --- | --- |
| MEM09002 | Interpret technical drawing |
| MEM11011 | Undertake manual handling |
| MEM12007 | Mark off/out structural fabrications and shapes |
| MEM12023 | Perform engineering measurements |
| MEM12024 | Perform computations |
| MEM13015 | Work safely and effectively in manufacturing and engineering |
| MEM14006 | Plan work activities |
| MEM16006 | Organise and communicate information |
| MEM18001 | Use hand tools |
| MEM18002 | Use power tools/hand held operations |

# Competency Field

Marine craft construction

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe the performance needed to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Determine job requirements | 1.1 | Follow standard operating procedures (SOPs) |
|  |  | 1.2 | Comply with work health and safety (WHS) requirements at all times |
|  |  | 1.3 | Use appropriate personal protective equipment (PPE) in accordance with SOPs |
|  |  | 1.4 | Identify job requirements from specifications, drawings, job sheets or work instructions |
| 2 | Construct marine vessel timber components | 2.1 | Select appropriate tools/equipment to suit job application |
|  |  | 2.2 | Select template/measurement to suit component construction |
|  |  | 2.3 | Ensure material is dressed, sized and shaped to suit job requirements and specifications |
|  |  | 2.4 | Check final form/shape of component against template/measurement specification and modify, as necessary |
|  |  | 2.5 | Apply component identification and set-up markings |
| 3 | Assemble components | 3.1 | Set up jig support assembly to suit job requirements |
|  |  | 3.2 | Select materials for fastening, joining and sealing and use according to job specifications and requirements |
|  |  | 3.3 | Determine assembly and fastening sequence |
|  |  | 3.4 | Carry out material preservation requirements, as required |
|  |  | 3.5 | Assemble components to specified line markings and positions |
|  |  | 3.6 | Ensure assembled component meets specifications |
|  |  | 3.7 | Clean/prepare component assembly to suit job requirements |

# Foundation Skills

This section describes those required skills (reading, writing, oral communication and numeracy) that are essential to workplace performance in this unit of competency.

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Range of Conditions

| This field allows for different work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) are included. |  |
| --- | --- |
| Marine vessel backbone components include one (1) or more of the following: | keel deadwoods stem transverse and longitudinal framing knees deck framing hull construction laid deck construction superstructures |
| Materials include one (1) or more of the following: | hardwood softwood plywood sheeting a combination |
| Tools and equipment include one (1) or more of the following: | hand and hand held power tools workshop equipment bandsaw surfacer thicknesser |
| Fastening methods include one (1) or more of the following: | marine-grade adhesives sealant and caulking materials metal fasteners (copper/silicon bronze/monel/stainless steel) |

# Unit Mapping Information

Release 2. Supersedes and is equivalent to MEM25005 Construct and assemble marine vessel timber components (Release 1).

Release 1. Supersedes and is equivalent to MEM25005B Construct and assemble marine vessel timber components.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b7050d37-5fd0-4740-8f7d-3b7a49c10bb2

Assessment Requirements for MEM25005 Construct and assemble marine vessel timber components

# Modification History

Release 2. Quantum of hours of workplace practice removed. Supersedes and is equivalent to MEM25005 Construct and assemble marine vessel timber components (Release 1).

Release 1. Supersedes and is equivalent to MEM25005B Construct and assemble marine vessel timber components.

# Performance Evidence

Evidence required to demonstrate competence in this unit must be relevant to and satisfy the requirements of the elements and performance criteria on at least two (2) occasions and include:

following work instructions, standard operating procedures (SOPs) and safe work practices

identifying and interpreting drawings, procedures, templates, construction details, assembly practices and material types to construct and assemble marine vessel timber components

cutting/sizing and preparing and forming/shaping timber against template/measurement specifications and modifying as required and checking final form/shape against job specifications for compliance

assembling supporting jigs to suit job requirements

selecting and using fastening methods and determining fastening sequence

finishing and applying preservation materials, cleaning excess material wastage and undertaking disposal of waste in accordance with SOPs.

# Knowledge Evidence

Evidence required to demonstrate the required knowledge for this unit must be relevant to and satisfy the requirements of the elements and performance criteria and include knowledge of:

safe work practices and procedures and use of personal protective equipment (PPE)

component types/materials/templates and their uses

characteristics, properties, specifications and uses of species, including durability, strength and appearance

types, specification and uses of fasteners

applicable codes and regulations

methods used to take off measurements, mark out, construct and assemble components

effect on access, rigidity, strength, buoyancy and resistance to stresses encountered at sea

hand tools, power tools and workshop machinery to suit construction applications

practices for cutting/dressing/sizing/shaping material

measuring and marking out techniques using graduated devices, battens and templates

procedures used in checking final form/shape

clamping and shoring methods and the use of jigs

procedures, codes and regulations used in applying relevant fastenings to suit assembly

methods of cleaning, preserving and finishing components, their uses and safety precautions

waste disposal obligations and regulations.

# Assessment Conditions

Assessors must:

have vocational competency in constructing and assembling marine vessel timber components at least to the level being assessed with relevant industry knowledge and experience

satisfy the assessor requirements in the Standards for Registered Training Organisations 2015 or its replacement and comply with the National Vocational Education and Training Regulator Act 2011, its replacement or equivalent legislation covering VET regulation in a non-referring state/territory as the case requires.

Assessment must occur in a functioning workplace. Where assessment in the workplace would be unsafe, impractical or threatens the environment, assessment must occur in a sufficiently rigorous simulated environment that reflects the circumstances that would be experienced in a functioning workplace. Assessment must cover all aspects of workplace performance, including task skills, task management skills, contingency management skills and job role environment skills.

Conditions for assessment must include access to all tools, equipment, materials and documentation required, including relevant workplace procedures, product and manufacturing specifications.

Assessment processes and techniques must be appropriate to the language, literacy and numeracy requirements of the work being performed and the needs of the candidate.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b7050d37-5fd0-4740-8f7d-3b7a49c10bb2