# MEM26006 Mark and cut out sheets for composite use

Release: 1

MEM26006 Mark and cut out sheets for composite use

# Modification History

Release 1. Supersedes and is equivalent to MEM26006A Mark and cut sheets for composite use

# Application

This unit of competency defines the skills and knowledge required to mark and cut out sheets for composite use.

The marking out/cutting out of sheets used for composites can be significantly different to the marking out/cutting out of other materials, including metals. Sheets used in composite fabrication often have significant directionality which must be considered and as a result of this composite design often includes different physical properties in one direction to another and so sheets must be marked and cut to align the required directional properties of the product with those of the sheets used in the fabrication.

Where the selection and use of hand tools is required unit MEM18001 Use hand tools should also be selected.

Where the selection and use of power tools is required unit MEM18002 Use power tools/hand held operations should also be selected.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

Band:	A

Unit Weight:	4

# Pre-requisite Unit

| MEM13015 | Work safely and effectively in manufacturing and engineering |
| --- | --- |
| MEM16006 | Organise and communicate information |

# Competency Field

Composites

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe the performance needed to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Determine product requirements | 1.1 | Follow standard operating procedures (SOPs) |
|  |  | 1.2 | Comply with work health and safety (WHS) requirements at all times |
|  |  | 1.3 | Use appropriate personal protective equipment (PPE) in accordance with SOPs |
|  |  | 1.4 | Determine requirements of sheet material to be used from specifications, job sheets or work instructions and customer requests |
|  |  | 1.5 | Identify process to be used to fabricate product |
| 2 | Develop geometric shape | 2.1 | Develop lay flats for regular solid rectilinear shapes |
|  |  | 2.2 | Develop lay flats for regular curved/circular shapes |
|  |  | 2.3 | Develop lay flats for transition pieces |
|  |  | 2.4 | Develop lay flats for complex 3-D shapes |
| 3 | Orient required shape to suit directional differences of sheet | 3.1 | Identify directional properties in sheet |
|  |  | 3.2 | Identify sheet-related directional requirements of product |
|  |  | 3.3 | Identify orientations and alignments of lay flats with sheet |
| 4 | Mark out and cut sheet | 4.1 | Identify and control hazards |
|  |  | 4.2 | Place lay flat on sheet and arrange lay flats to minimise waste |
|  |  | 4.3 | Prevent lay flat moving and mark sheet, as required, using lay flat |
|  |  | 4.4 | Cut sheet to meet requirements |
|  |  | 4.5 | Identify typical problems and take appropriate remedial action |
|  |  | 4.6 | Perform final verification to meet product requirements |
| 5 | Store templates/lay flats | 5.1 | Clean lay flats after use |
|  |  | 5.2 | Label lay flat using identification protocol |
|  |  | 5.3 | Record required directionality information with lay flat |
|  |  | 5.4 | Store lay flats according to procedures |
|  |  | 5.5 | Complete logs and reports according to SOPs |

# Foundation Skills

This section describes those required skills (reading, writing, oral communication and numeracy) that are essential to workplace performance in this unit of competency.

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Range of Conditions

| This field allows for different work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) are included. |  |
| --- | --- |
| Sheets include one (1) or more of the following: | reinforcing: woven non-woven pre-impregnated (pre-preg) cores timber, including medium density fibreboard (MDF) and plywood filler material |
| Requirements of sheet material includes the following: | strength flexibility/rigidity thickness density directionality drape operating temperature |
| Regular rectilinear shapes include one (1) or more of the following: | rectangular prisms cubes portions of prisms and cubes similar shapes |
| Regular curved shapes include one (1) or more of the following: | cylinders spheres segments of cylinders and spheres similar shapes |
| Transition pieces include one (1) or more of the following: | cones pyramids portions of cones and pyramids square to round tee piece lobster back similar shapes |
| Complex 3-D shapes include the following: | other 3-D shapes which may need to be fabricated and which may be composed of a number of components of the above shapes |
| Alignment includes the following: | warp weft selvedge other features of the sheet the nesting of lay flats to optimise the use of materials and minimise waste |
| Typical problems include one (1) or more of the following: | identifying directionality in sheet identify a feature to provide alignment products requiring multidirectional properties |

# Unit Mapping Information

Release 1. Supersedes and is equivalent to MEM26006A Mark and cut sheets for composite use

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b7050d37-5fd0-4740-8f7d-3b7a49c10bb2

Assessment Requirements for MEM26006 Mark and cut out sheets for composite use

# Modification History

Release 1. Supersedes and is equivalent to MEM26006A Mark and cut sheets for composite use

# Performance Evidence

Evidence required to demonstrate competence in this unit must be relevant to and satisfy the requirements of the elements and performance criteria on at least two (2) occasions and include:

following work instructions, standard operating procedures (SOPs) and safe work practices

identifying and interpreting specifications, formulas/recipes, batch sheets, product drawings, customer requests for marking and cutting sheets for composite use

developing lay flats for geometric shapes

marking out and cutting sheets taking into account the directional properties and alignments of lay flats and minimising waste

identifying typical problems and taking appropriate remedial action

cleaning, labelling, recording directionality information and storing lay flats in accordance with procedures

completing all documentation and reports, including items requiring further action in accordance with SOPs.

# Knowledge Evidence

Evidence required to demonstrate the required knowledge for this unit must be relevant to and satisfy the requirements of the elements and performance criteria and include knowledge of:

safe work practices and procedures and use of personal protective equipment (PPE)

current state/territory work health and safety (WHS) legislation, standards and codes of practice relevant to marking and cutting sheets for composite use

orientation of plies/reinforcing

direction of fibres (e.g. unidirectional, quad directional and on bias)

weaves (e.g. plain, twill and satin)

use for curves and compound curves

creating patterns for a lay flat

3-D development of shapes.

# Assessment Conditions

Assessors must:

have vocational competency in marking and cutting sheets for composite use at least to the level being assessed with relevant industry knowledge and experience

satisfy the assessor requirements in the Standards for Registered Training Organisations 2015 or its replacement and comply with the National Vocational Education and Training Regulator Act 2011, its replacement or equivalent legislation covering VET regulation in a non-referring state/territory as the case requires

Where possible assessment must occur in operational workplace situations. Where this is not possible or where personal safety or environmental damage are limiting factors, assessment must occur in a sufficiently rigorous simulated environment that reflects realistic operational workplace conditions. This must cover all aspects of workplace performance, including environment, task skills, task management skills, contingency management skills and job role environment skills

Conditions for assessment must include access to all tools, equipment, materials and documentation required, including relevant workplace procedures, product and manufacturing specifications

Assessment processes and techniques must be appropriate to the language, literacy and numeracy requirements of the work being performed and the needs of the candidate.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b7050d37-5fd0-4740-8f7d-3b7a49c10bb2