# MEM29013 Integrate sensors into digital manufacturing processes

Release: 1

MEM29013 Integrate sensors into digital manufacturing processes

# Modification History

Not applicable.

# Application

This unit defines the skills and knowledge required to prepare, configure and test sensors for integration into other digital advanced manufacturing systems and technologies including those covered under the term Industry 4.0. The term sensor is used in this unit for devices that receive a stimulus signal and convert the signal to an electrical signal either within the sensor (i.e. a transducer) or with the aid of an external processor. The term transducer is only used where the distinction between onboard and external processing is important in the unit. Sensors covered by the skills and knowledge in the unit are often referred to as smart sensors and transducers and are usually wireless, distributed, and networked.

The unit applies to individuals responsible for integrating smart sensors with other Industry 4.0 systems and processes to ensure effective data identification, communication, and processing as well as monitoring and control of physical processes. The unit does not include mechanical or electronic maintenance or repair of sensors or connection to electricity supply levels that require an electrical licence.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil.

# Competency Field

Applied technologies

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Select appropriate sensor for a required application | 1.1 Identify workplace health and safety (WHS) procedures and risk control measures relevant to the selection of sensors in manufacturing and engineering 1.2 Identify and review data required to be gathered by sensor 1.3 Identify optimal location for sensor 1.4 Review up and downstream sensor communication requirements 1.5 Identify if on board, next stage, or remote processing of sensor or transducer data is required 1.6 Select optimal sensor connectivity option 1.7 Select optimal sensor for location and data collection requirements |
| 2. Identify opportunities for using sensors and transducers to optimise Industry 4.0 related systems | 2.1 Recommend sensor strategy for improved process monitoring 2.2 Recommend sensor strategy for improved predictive and preventative maintenance 2.3 Recommend sensor strategy for improved use of robotics and automation 2.4 Recommend sensor strategy for improved quality management 2.5 Recommend sensor strategy for improved waste control |
| 3. Install and configure sensors | 3.1 Mount sensor according to manufacturer’s instructions and workplace requirements 3.2 Install shielding or case in accordance with manufacturer’s specifications 3.3 Install and test connections and interfacing in accordance with manufacturer’s specifications 3.4 Calibrate sensor according to manufacturer’s instructions and workplace requirements |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential to performance.

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Range of Conditions

| This field allows for different work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) are included. |  |
| --- | --- |
| Industry 4.0 related systems include one or more of the following: | industrial internet of things (IIoT) devices collaborative robots networked and cloud connected traditional robots and other industrial automation networked and cloud connected supervisory control and data acquisition systems (SCADA) networked and cloud connected enterprise resource planning (ERP) systems networked and cloud connected CAD and CAM systems digital twins augmented and virtual reality systems networked and cloud connected CNC and additive manufacturing machines networked and cloud connected data storage and processing facilities networked and cloud connected edge devices other networked and cloud connected digital devices and systems relevant to the workplace. |
| Physical properties measured by sensors include one or more of: | temperature pressure position weight vibration gas and liquid flow location proximity movement state/phase conductivity magnetism other physical properties. |

# Unit Mapping Information

No equivalent unit

# Links

Companion Volume Implementation Guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b7050d37-5fd0-4740-8f7d-3b7a49c10bb2

Assessment Requirements for MEM29013 Integrate sensors into digital manufacturing processes

# Modification History

Release 1. New unit.

# Performance Evidence

There must be evidence the candidate has demonstrated the ability to:

report on the current use of sensors in the workplace including descriptions of:

types and locations of sensors

physical properties measured by the installed sensors

range of data collected

sensor data transmission, processing and networking

report on the opportunity to increase the amount and quality of collected data through:

use of additional sensor locations and types

use of intelligent sensors including sensors incorporating self-testing and self-calibration

use of feedback loops and edge processing of sensor data

read logic flow and state diagrams

install and correctly calibrate four different types of sensors including:

installing and calibrating at least one transducer,

connecting at least two sensors to a 5G network

following networking protocols and interfacing standards for installing sensors including:

input/output (I/O) – digital and analog

handshaking.

Note: Where a volume and/or frequency is not specified, demonstration must be provided at least once.

# Knowledge Evidence

There must be evidence the candidate has knowledge of:

sensors in a smart factory or workplace and their contribution to:

providing data for visibility on processes, materials and equipment

issue identification and troubleshooting

reliability including preventative and predictive maintenance

sensors and transducers as sources of data for different levels in an organisation including:

process or equipment control

operations management and quality control

predictive and preventative maintenance

fault diagnosis

data analytics for economic gain

similarities and differences between a sensor and a transducer including:

sensors and transducers as devices that detect changes in their surrounding environment or the object to which they are attached

a sensor gives output in the same format as its input information

a transducer converts the input signal into an electrical signal

principles for classifying sensors including:

output signal –

energy source - active (requiring external power) and passive (no power source required)

contact and non-contact with input stimulus

type of output measurement - absolute or relative measurement of input stimulus

other classifications

characteristics to consider when selecting sensors including:

robustness including:

sensitivity to interference

shielding/housing needs

environmental limits– dust, moisture, temperature etc.

interfacing and signal conditioning

networking including:

5G

WiFi

wired

cloud

two-way communication

data processing requirements including:

on board processing

adjacent or edge processor

network or cloud-based processor

location requirements including:

physical connections

positioning to maximise gathering of information

current and voltage

output devices/ actuators

cost

status register functions: carry©, Digit Carry (DC), Zero (Z) bits

accuracy – measured value versus true value

sensor saturation or threshold point

developments in sensor technology including awareness of the following and their potential application:

nano sensors

micro sensors

nuclear sensors

# Assessment Conditions

Assessors must:

have vocational competency in integrating sensors into digital manufacturing processes at least to the level being assessed with relevant industry knowledge and experience

satisfy the assessor requirements in the Standards for Registered Training Organisations 2015 or its replacement and comply with the National Vocational Education and Training Regulator Act 2011, its replacement or equivalent legislation covering VET regulation in a non-referring state/territory as the case requires.

Where possible, assessment must occur in operational workplace situations. Where this is not possible or where personal safety or environmental damage are limiting factors, assessment must occur in a sufficiently rigorous simulated environment that reflects realistic operational workplace conditions that cover all aspects of workplace performance, including environment, task skills, task management skills, contingency management skills and job role environment skills.

There must be access to all tools, equipment, materials and documentation required, including relevant workplace procedures, product and manufacturing specifications.

Assessment processes and techniques must be appropriate to the language, literacy and numeracy requirements of the work being performed and the needs of the candidate.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=b7050d37-5fd0-4740-8f7d-3b7a49c10bb2