# MSFBA2001 Use blinds, awnings, security screens and grilles hand and power tools

Release: 1

MSFBA2001 Use blinds, awnings, security screens and grilles hand and power tools

# Modification History

Release 1 - New unit of competency

# Application

This unit of competency covers using hand and power tools in applications relating to the blinds and awnings, and security screens and grilles sectors of the furnishing industry. It applies in workplaces and on-site environments and may be conducted in small to large scale enterprises.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

# Competency Field

# Unit Sector

Blinds and awnings, security screens and grilles

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe the performance needed to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Identify hand and power tools | 1.1 | Types of hand and power tools and their functions are identified |
|  |  | 1.2 | Sources of power supply are recognised |
|  |  | 1.3 | Work health and safety (WHS), legislative requirements and workplace practices for using hand and power tools are recognised and adhered to |
| 2 | Select hand tools | 2.1 | Appropriate personal protective equipment is selected, correctly fitted and used |
|  |  | 2.2 | Hand tools are selected consistent with the needs of the task |
|  |  | 2.3 | Tools are checked for safe operation and any faults are corrected or tagged and reported to appropriate personnel |
|  |  | 2.4 | Equipment is selected to hold or support material for hand tool operation |
| 3 | Use hand tools | 3.1 | Material is located and held in position for hand tool operation |
|  |  | 3.2 | Hand tools are held correctly for safe operation |
|  |  | 3.3 | Hand tools are used according to manufacturer instructions and safe work practices |
|  |  | 3.4 | Hand tools are safely located when not in immediate use |
| 4 | Select power tools | 4.1 | Power tools are selected consistent with the needs of the task |
|  |  | 4.2 | Power tools are checked for serviceability and safe operation in accordance with WHS requirements |
|  |  | 4.3 | Power tool faults are corrected or tagged and reported to appropriate personnel |
|  |  | 4.4 | Equipment is selected to hold or support materials for power tool operation |
| 5 | Use power tools | 5.1 | Material is held in position for power tool operation |
|  |  | 5.2 | Power tools are set up for use according to manufacturer instructions and safe working practices |
|  |  | 5.3 | Power tools are held correctly for safe and efficient use |
|  |  | 5.4 | Power tools are used according to manufacturer instructions and safe work practices |
|  |  | 5.5 | Power tools are safely located when not in immediate use |
| 6 | Clean up work area and tools | 6.1 | Hand and power tools are cleaned after use |
|  |  | 6.2 | Operator maintenance is carried out following completion of task. |
|  |  | 6.3 | Hand and power tools are stored following completion of task |
|  |  | 6.4 | Equipment is cleaned, maintained and stored |
|  |  | 6.5 | Work area is cleared and waste removed |
|  |  | 6.6 | Workplace documentation is completed |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency. Detail on appropriate performance levels for each furnishing unit of competency in reading, writing, oral communication and numeracy utilising the Australian Core Skills Framework (ACSF) are provided in the Furnishing Training Package Implementation Guide.

# Range of Conditions

| Specifies different work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) are included. Range is restricted to essential operating conditions and any other variables essential to the work environment. |  |
| --- | --- |
| Hand tools include: | calculators pop rivet gun screwdrivers combination saw panel saws hack saws socket set hand grinder hammers files tin snips spanners level square measuring tape hex keys caulking gun markers tensioning devices G clamps string lines scissors eyelet punches laser level stud finder pipe bender knives sheers measuring device straight edge rollers metal press tap and die sets |
| Power tools include: | bench grinders hammer drills cordless screwdriver/drill air drills jig saws drop saw docking saw circular saw press-stud machine hot knives staple gun hollow wall anchor gun planer |
| Power supply includes: | battery, mains, electricity and pneumatic |
| WHS requirements include: | Commonwealth, state or territory legislation and regulations organisational safety policies and procedures use of personal protective equipment firefighting equipment first aid equipment hazard and risk control and elimination of hazardous materials and substances manual handling, including lifting and carrying |
| Legislative requirements include: | applicable legislation from all levels of government that affect organisational operation award and enterprise agreements industrial relations Australian Standards confidentiality and privacy WHS the environment equal employment opportunity (EEO) anti-discrimination relevant industry codes of practice duty of care and heritage |
| Workplace practices include: | legal, organisational and site guidelines policies and procedures relating to own role and responsibility quality assurance procedural manuals quality and continuous improvement processes and standards emergency and evacuation ethical standards recording and reporting access and equity principles and practices equipment use maintenance and storage environmental management (waste disposal, recycling and re-use guidelines) tolerances following manufacturer instructions and specifications |
| Personal protective equipment includes: | safety glasses/goggles hearing protection safety footwear protective clothing and head wear respiratory protection |
| Appropriate personnel include: | trainer assessor supervisor team leader maintenance officer manager or any person responsible for the activities in the workplace/worksite |
| Equipment includes: | clamps, jigs, braces and ladders |
| Material includes: | textiles (canvas, acrylic, PVC, clear PVC, mesh and binding) metals (aluminium and steel) timber steel or aluminium material in extruded or sheet form synthetic, aluminium or steel screen material |
| Safe operation includes: | safe work practices followed in accordance with industry and enterprise specific policies and procedures |
| Waste includes: | textiles, metals, timber, metal off-cuts and shavings waste removal should be in accordance with enterprise policies and environmental legislation covering disposal of industrial waste |
| Workplace documentation includes: | time sheets customer care cards work orders and reports, such as machining method, product type, size, inspection, grading and labelling outcomes, storage locations, quality outcomes, hazards, and incidents or equipment malfunctions |

# Unit Mapping Information

Supersedes and is equivalent to LMFBA2001A Use blinds and awnings sector hand and power tools

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0601ab95-583a-4e93-b2d4-cfb27b03ed73

Assessment Requirements for MSFBA2001 Use blinds, awnings, security screens and grilles hand and power tools

# Modification History

Release 1 - New unit of competency

# Performance Evidence

Communicate with workplace/worksite supervisor to:

confirm work orders and instructions

identify problems

Apply safe handling practices for equipment, products and materials and minimise the risk of injury to self and others

Prevent damage to goods, tools, equipment or products and maintain required production output and product quality

Comply with legislation, regulations, standards, codes of practice and established safe practices and procedures for use of hand and power tools

Identify, select and safely use hand and power tools for manufacturing and fitting blinds, awnings, security screens and grilles in the workplace/worksite

Report outcomes of hand and power tool use and maintenance problems

Use pre-checking and inspection techniques to anticipate problems, avoid re-working and wastage

Participate in establishing new ideas and work specifications for selecting and using hand and power tools

Plan and organise own work, including the preparation and layout of the work area, and obtaining equipment and materials to avoid back tracking

# Knowledge Evidence

State/territory WHS legislation, regulations, standards and codes of practice relevant to hand and power tool use

Workplace and worksite, requirements, policies and procedures for hand and power tool use

Reading and interpreting workplace documents, including work orders, work instructions and operator instructions

Environmental protection requirements relating to the disposal of waste material

Established lines of communication and workplace/worksite protocols

Types of tools and equipment and procedures for their safe use, operation and maintenance

Procedures for the reporting, recording and maintenance of workplace records and information

Procedures for reporting and recording faulty hand and power tools

Characteristics of steel and/or aluminium, steel and/or aluminium products and defects

Industry standard cross-sections and lengths

Relevant storage systems and labelling

# Assessment Conditions

Assessors must:

hold training and assessment competencies as determined by the National Skills Standards Council (NSSC) or it successors

have vocational competency in the furnishing industry at least to the level being assessed with broad industry knowledge and experience, usually combined with a relevant industry qualification

be familiar with the current skills and knowledge used and have relevant, current experience in the furnishing industry.

Assessment methods must confirm consistency of performance over time rather than a single assessment event and in a range of workplace relevant contexts.

Assessment must be by observation of relevant tasks with questioning on underpinning knowledge and, where applicable, multimedia evidence, supervisor’s reports, projects and work samples.

Assessment is to be conducted on single units of competency or in conjunction with other related units of competency. Foundation skills are integral to competent performance in the unit and should not be assessed separately.

Assessment must occur on the job or in a workplace simulated facility with relevant process, equipment, materials, work instructions and deadlines.

Access is required to a location, and standard operating procedures for using blinds, awnings security screens and grilles hand and power tools relevant to manufacturing or fitting blinds, awnings, security screens and grilles.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0601ab95-583a-4e93-b2d4-cfb27b03ed73