# MSFBA2012 Use blinds, awnings, security screens and grilles hand and power tools

Release: 1

MSFBA2012 Use blinds, awnings, security screens and grilles hand and power tools

# Modification History

Release 1. Changes to code, application, elements, performance criteria. Foundation skills added. Range of conditions removed. Assessment requirements changed.

# Application

This unit describes the skills and knowledge required to select and use hand and power tools to manufacture and install blinds, awnings, security screens and grilles. Workers with this competency use tools to complete routine tasks.

No licensing or certification requirements exist at the time of publication. Relevant legislation, industry standards and codes of practice within Australia must be applied.

# Competency Field

Blinds and awnings

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Prepare for tool use | 1.1 Identify task requirement from work instructions 1.2 Select the appropriate tool based on task requirements and identify work health and safety requirements for its use 1.3 Confirm serviceability and safety of tool and report any faults to supervisor 1.4 Select equipment to hold or support material based on the tool selected and the nature of the task 1.5 Prepare material and hold in position to ensure safe operation |
| 2. Use tools | 2.1 Use tools safely and in accordance with workplace procedures and manufacturer specifications 2.2 Use techniques that optimise the effective use of the tool for the nominated task 2.3 Ensure safe state, position and location of tools in the work area when not in immediate use 2.4 Follow safety procedures for the use of power and the handling of power tools |
| 3. Clean up work area and tools | 3.1 Clean tools and equipment using cleaning materials and agents suited to the item 3.2 Complete routine operator maintenance on tools in accordance with workplace procedures and manufacturer instructions 3.3 Store tools and equipment based on specific storage needs and workplace requirements |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential to performance but not explicit in the performance criteria.

Reading skills to interpret familiar workplace procedures

Numeracy skills to interpret work instructions associated with number or measurement.

Other foundation skills essential to performance are explicit in the performance criteria of this unit.

# Unit Mapping Information

Supersedes and is equivalent to MSFBA2001 Use blinds, awnings security screens and grilles sector hand and power tools

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0601ab95-583a-4e93-b2d4-cfb27b03ed73