# Assessment Requirements for MSFFDM4006 Hand carve wood to custom design

Release: 1

Assessment Requirements for MSFFDM4006 Hand carve wood to custom design

# Modification History

Release 1. Unit code and Application changed. Elements and Performance Criteria changed to active voice. Range of Conditions removed and relevant content moved to Assessment Requirements. Assessment Requirements changed to remove duplication with Performance Criteria and clarify requirements. Supersedes and is equivalent to MSFFM4001 Hand carve wood to custom design.

# Performance Evidence

There must be evidence the candidate has completed the tasks outlined in the elements and performance criteria of this unit, and demonstrated the ability to:

hand carve two significant products, showing:

the interpretation and necessary adoption of the design specifications

the selection of suitable timber for the carving

the completion of the carving to specification or commercially acceptable standard

the preparation of the products for finishing

carry out operator maintenance on hand tools used across the two significant products, including the grinding and sharpening of at least one knife and one chisel.

# Knowledge Evidence

There must be evidence the candidate has knowledge of:

terminology covering hand carving operations and materials

techniques used in hand carving, including:

chip carving

relief carving

Scandinavian flat plane

caricature carving

love spoon

treen

whittling

types, characteristics, uses and limitations of timber species for hand carving

techniques for the design and documentation of specifications for hand carved products

workplace and site standards, requirements, policies and procedures for hand carving of wood

types, characteristics, uses, limitations and maintenance requirements of the range of tools used in wood carving, including:

carving knives

chisels (straight and curved)

short bent front bent

spoon bit

back bent

skew

fishtail

spade

parting tool

macaroni (fluterino and backeroni)

palm tools

rasps

rifflers

punches

gouges

venier

router

processes for evaluating the quality of carving tools in terms of specifications, safety, maintainability and commercial reputation

preparation methods wood for carving, including:

confirming grain

identifying and responding to faults

dealing with knots and contaminants

cutting wood to workable size and shape

applicable work health and safety, legislative and workplace requirements and instructions relevant to the hand carving of wood.

# Assessment Conditions

Skills must have been demonstrated in the workplace or in a simulated environment that reflects workplace conditions and contingencies. The following conditions must be met for this unit:

use of suitable facilities, equipment and resources, including:

product specifications and/or drawings

a range of hand carving tools.

Assessors must satisfy the NVR/AQTF mandatory competency requirements for assessors.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0601ab95-583a-4e93-b2d4-cfb27b03ed73