# Assessment Requirements for MSFFDM4013 Produce curved and shaped components for custom furniture

Release: 1

Assessment Requirements for MSFFDM4013 Produce curved and shaped components for custom furniture

# Modification History

Release 1. Unit code and Application changed. Elements and Performance Criteria changed to active voice. Range of Conditions removed and relevant content moved to Assessment Requirements. Assessment Requirements changed to include volume and frequency of evidence and remove duplication with Performance Criteria. Supersedes and is equivalent to MSFFM4003 Produce curved and shaped components for custom furniture.

# Performance Evidence

There must be evidence the candidate has completed the tasks outlined in the elements and performance criteria of this unit, and demonstrated the ability to:

produce six significant timber curved components, five from solid timber and one using ply:

one involving shaping from solid timber using hand techniques

one involving shaping from solid timber using machine techniques

one involving laminating

one involving use of heat/steam

one involving other techniques

items must demonstrate appropriate design, use of materials and strength for purpose

perform checks on tools and equipment used across the six components, relating to appropriateness for use, service standards and safety.

# Knowledge Evidence

There must be evidence the candidate has knowledge of:

techniques for timber bending, forming and shaping, including:

laminating

coopering

steam bending

saw kerfing

hand shaping

machine shaping

timber shaped by hand or static machine tools

types and characteristics of materials use in the curving or shaping of components, including:

solid timber and timber composites including plywood, medium density fibreboard (MDF) and veneer

plastic laminate

sheet metal

adhesives

screws

nails

dowels

decorative finishes

abrasive paper

finishing materials

types, characteristics, uses, limitations and maintenance requirements of solid timbers commonly used in furniture production

types, characteristics, uses and limitations on hand and power tools used in timber bending, forming and shaping, including:

band, cross-cut, mitre, panel and rip saws

surface and panel planers

belt sanders

horizontal borers

vertical drill presses

dove-tailers

pedestal grinders

wood-turning lathes

veneer guillotines

presses

interpretation of plan representations of furniture design

techniques for the preparation of drawings/set-outs/rods and geometrical concepts

workflow within the workplace

applicable work health and safety, legislative and workplace requirements relevant to the production of curved and shaped components for custom furniture are verified and complied with

workplace and site standards, requirements, policies and procedures for the production of curved and shaped components for custom furniture

# Assessment Conditions

Skills must have been demonstrated in the workplace or in a simulated environment that reflects workplace conditions and contingencies. The following conditions must be met for this unit:

use of suitable facilities, equipment and resources, including:

hand and power tools

a wide range of timber and timber composite materials

Assessors must satisfy the NVR/AQTF mandatory competency requirements for assessors.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0601ab95-583a-4e93-b2d4-cfb27b03ed73