# Assessment Requirements for MSFFF2014 Apply surface coatings by spray gun

Release: 1

Assessment Requirements for MSFFF2014 Apply surface coatings by spray gun

# Modification History

Release 1. Changes to code, application, elements, performance criteria, assessment requirements. Foundation skills added. Range of conditions removed. Assessment requirements changed.

# Performance Evidence

There must be evidence the candidate has completed all the tasks outlined in the elements and performance criteria of this unit, and demonstrated the ability to:

select and apply surface coatings, including the adjustment of spray equipment to achieve required spray pattern, on at least 3 occasions so that the coatings are free of faults, with a consistent sheen, and meet workplace quality standards. At least 2 of the following surface types must be included:

solid timber

manufactured board

veneer board

metal.

# Knowledge Evidence

There must be evidence the candidate has knowledge of:

types of spray equipment, including:

conventional air spray guns and airless spray guns

high volume/low pressure (HVLP) air-assisted and airless spray guns

spray processes

types of coating, including:

pre-catalysed lacquer

acid catalysed lacquer and nitro-cellulose lacquer

one-pot polyurethane and water-based coatings

stains, lacquers, polish, enamels and acrylics

methods to prevent contamination of surfaces during and after surface coating

workflow for spraying operations

the function of tools and equipment commonly used in applying spray coatings, including at least 6 of the following:

pressure feed systems

spray booth

liquid containers

air compressor and hoses

air regulator

positive pressure air-wash masks

canister-type face masks

screwdrivers

shifting spanner and spanners

sources of workplace information, including:

workplace procedures relating to the use of tools and equipment

work instructions

workplace procedures relating to reporting and communication

manufacturer specifications and operational procedures

work health and safety requirements, including manual handling procedures and the use of personal protective equipment (PPE).

# Assessment Conditions

Skills must have been demonstrated in the workplace or in a simulated environment that reflects workplace conditions and contingencies. The following conditions must be met for this unit:

use of suitable facilities, equipment and resources, including:

work orders

items requiring spray coatings

spray equipment, spray booth, coatings and cleaning products required for performance evidence

PPE

workplace procedures, manufacturing specifications, codes, standards, manuals, and reference materials.

Assessors must satisfy the NVR/AQTF mandatory competency requirements for assessors.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0601ab95-583a-4e93-b2d4-cfb27b03ed73