# MSFFF2014 Apply surface coatings by spray gun

Release: 1

MSFFF2014 Apply surface coatings by spray gun

# Modification History

Release 1. Changes to code, application, elements, performance criteria, assessment requirements. Foundation skills added. Range of conditions removed. Assessment requirements changed.

# Application

This unit describes the skills and knowledge required to apply surface coatings using a handheld spray gun. Workers with this competency complete routine tasks in the furniture finishing industry.

No licensing or certification requirements exist at the time of publication. Relevant legislation, industry standards and codes of practice within Australia must be applied.

# Competency Field

Furniture finishing

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Plan and prepare for task | 1.1 Determine task requirements, including characteristics of surface and required surface coating, from work documentation 1.2 Plan task by identifying processing and application techniques, time and safety requirements and efficient work methods 1.3 Identify and follow work health and safety requirements 1.4 Identify tools, equipment and accessories to meet task requirements and check they are safe and operational 1.5 Identify and prepare required surface coatings and other materials and check they meet task quality requirements |
| 2. Check and prepare for spraying | 2.1 Identify surface or other faults in products to be coated and report or repair faults in accordance with workplace procedures 2.2 Protect other products and equipment in the workplace from overspray 2.3 Perform trial applications of surface materials to check condition of spray system, spray patterns, equipment operation, materials viscosity and conformity with specified surface finish 2.4 Clean or replace unserviceable components and reassemble spray system 2.5 Identify and rectify spray pattern faults in accordance with workplace procedures. 2.6 Identify and rectify material faults in accordance with workplace procedures |
| 3. Apply surface coating | 3.1 Ensure surfaces are kept free of contamination 3.2 Apply surface coatings in accordance with workplace procedures 3.3 Check surface build and coverage against workplace requirements for even spread and thickness 3.4 Rectify surface faults in accordance with workplace procedures 3.5 Inspect products and approve for suitability for further processing |
| 4. Clean work area | 4.1 Check, clean and store equipment and unused materials in accordance with equipment instructions and workplace procedures 4.2 Tag unserviceable equipment, identify faults and inform appropriate personnel in accordance with workplace procedures 4.3 Clean work area, including spray booth, safely dispose of waste and identify items for recycling in accordance with workplace procedures and environmental standards 4.4 Complete required workplace records |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential to performance but not explicit in the performance criteria.

Reading skills to interpret familiar workplace procedures and work documentation

Writing skills to communicate simple information in familiar workplace documents

Numeracy skills to measure and calculate area and estimate material requirements.

Other foundation skills essential to performance are explicit in the performance criteria of this unit.

# Unit Mapping Information

Release 1. Supersedes and is equivalent to MSFFF2006 Apply surface coatings by spray gun.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0601ab95-583a-4e93-b2d4-cfb27b03ed73

Assessment Requirements for MSFFF2014 Apply surface coatings by spray gun

# Modification History

Release 1. Changes to code, application, elements, performance criteria, assessment requirements. Foundation skills added. Range of conditions removed. Assessment requirements changed.

# Performance Evidence

There must be evidence the candidate has completed all the tasks outlined in the elements and performance criteria of this unit, and demonstrated the ability to:

select and apply surface coatings, including the adjustment of spray equipment to achieve required spray pattern, on at least 3 occasions so that the coatings are free of faults, with a consistent sheen, and meet workplace quality standards. At least 2 of the following surface types must be included:

solid timber

manufactured board

veneer board

metal.

# Knowledge Evidence

There must be evidence the candidate has knowledge of:

types of spray equipment, including:

conventional air spray guns and airless spray guns

high volume/low pressure (HVLP) air-assisted and airless spray guns

spray processes

types of coating, including:

pre-catalysed lacquer

acid catalysed lacquer and nitro-cellulose lacquer

one-pot polyurethane and water-based coatings

stains, lacquers, polish, enamels and acrylics

methods to prevent contamination of surfaces during and after surface coating

workflow for spraying operations

the function of tools and equipment commonly used in applying spray coatings, including at least 6 of the following:

pressure feed systems

spray booth

liquid containers

air compressor and hoses

air regulator

positive pressure air-wash masks

canister-type face masks

screwdrivers

shifting spanner and spanners

sources of workplace information, including:

workplace procedures relating to the use of tools and equipment

work instructions

workplace procedures relating to reporting and communication

manufacturer specifications and operational procedures

work health and safety requirements, including manual handling procedures and the use of personal protective equipment (PPE).

# Assessment Conditions

Skills must have been demonstrated in the workplace or in a simulated environment that reflects workplace conditions and contingencies. The following conditions must be met for this unit:

use of suitable facilities, equipment and resources, including:

work orders

items requiring spray coatings

spray equipment, spray booth, coatings and cleaning products required for performance evidence

PPE

workplace procedures, manufacturing specifications, codes, standards, manuals, and reference materials.

Assessors must satisfy the NVR/AQTF mandatory competency requirements for assessors.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0601ab95-583a-4e93-b2d4-cfb27b03ed73