# MSFFL2031 Remove existing floor coverings

Release: 1

MSFFL2031 Remove existing floor coverings

# Modification History

Release 1. Supersedes and is equivalent to MSFFL2005 Remove existing floor coverings.

# Application

This unit describes the skills and knowledge required to remove existing floor coverings.

This unit applies to individuals removing floor coverings in accordance with established workplace procedures.

Licensing, legislative or certification requirements may apply to this unit and relevant state/territory and local government agencies should be consulted to determine any necessary certification or licensing for undertaking flooring work. Industry standards and codes of practice within Australia must also be applied. Access to construction sites requires certification of general induction training specified by the National Code of Practice for Induction for Construction Work (ASCC 2007).

# Pre-requisite Unit

N/A

# Unit Sector

Flooring technology

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Plan work | 1.1 Identify and follow work health and safety, legislative and organisational requirements 1.2 Read documentation and identify work requirements 1.3 Select tools, equipment and materials and check for suitability, serviceability and safety prior to use 1.4 Plan sequence of work to ensure removal of floor coverings is in logical order 1.5 Prepare and position materials and equipment for use |
| 2. Remove floor coverings | 2.1 Select starting point to ensure entry and exit access throughout the operation 2.2 Check area and flooring for presence of any hazardous substances and take appropriate action 2.3 Remove floor covering using techniques suited to the type of material 2.4 Securely package removed floor coverings in manageable bundles for removal from the area 2.5 Check exposed floor and safely remove any protruding nails, carpet grippers, adhesives, levelling compounds and remnants of floor coverings |
| 3. Complete housekeeping | 3.1 Store and recycle unused materials in accordance with workplace procedures 3.2 Check, clean and store tools and equipment in accordance with workplace procedures 3.3 Clean work area and dispose of rubbish safely 3.4 Accurately complete required workplace documentation |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Unit Mapping Information

Supersedes and is equivalent to MSFFL2005 Remove existing floor coverings.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0601ab95-583a-4e93-b2d4-cfb27b03ed73