# Assessment Requirements for MSFFL2034 Prepare subfloors with cementitious coating applications

Release: 1

Assessment Requirements for MSFFL2034 Prepare subfloors with cementitious coating applications

# Modification History

Release 1. Supersedes and is not equivalent to MSFFL2006 Prepare, select and apply smoothing and patching compounds, and, MSFFL2007 Select and apply appropriate compounds and additives.

# Performance Evidence

There must be evidence the candidate has completed the tasks outlined in the elements and performance criteria of this unit, and:

achieved the required substrate finish with each of the following on both timber and concrete subfloors:

level coat to 3mm thickness

primer

rapid set mortar patch or ramp

skim coat.

# Knowledge Evidence

There must be evidence the candidate has knowledge of:

mathematical procedures for estimation and measurement, including calculation of area and estimation of material requirements

characteristics and requirements of concrete and timber subfloor preparation

characteristics and requirements of concrete and fibre board wall preparation

application of compounds and additives:

workplace safety system requirements

environmental protection and sustainability requirements

how to access manufacturer instructions

types, characteristics, uses and limitations of compounds and additives, including:

porous substrate primer

non-porous substrate primer

substrate moisture barrier and green slab primer

fibre

high polymer cement-based finishes

latex powder and emulsion based subfloor smoothing compounds

levelling and smoothing compounds and primers

pumpable cement base floor leveller

rapid setting and drying repair mortar

self-smoothing and/or levelling cements

synthetic resin dispersion

two part, chemically cured epoxy coating for porous concrete floors

graded aggregate for bulking

kiln dry sand for priming and bulking

types, characteristics, safe use, maintenance and limitations of tools, equipment and materials

techniques for the application of compounds and additives, the types of problems that may occur, and how to avoid and respond

quality indicators for subfloor surface finishes

procedures for the recording, reporting and maintenance of workplace records and information.

# Assessment Conditions

Skills must have been demonstrated in the workplace or in a simulated environment that reflects workplace conditions and contingencies. The following conditions must be met for this unit:

use of suitable facilities, equipment and resources, including:

subfloors requiring application of compounds and additives

compounds and additives

equipment, tools and materials to complete tasks in the Performance Evidence

work orders

workplace procedures.

Assessors must satisfy the NVR/AQTF mandatory competency requirements for assessors.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0601ab95-583a-4e93-b2d4-cfb27b03ed73