# Assessment Requirements for MSFFL2035 Mechanically prepare surfaces for installation of flooring materials

Release: 1

Assessment Requirements for MSFFL2035 Mechanically prepare surfaces for installation of flooring materials

# Modification History

Release 1. Supersedes and is not equivalent to:

MSFFL2011 Select, operate and maintain grinding equipment

MSFFL2012 Select, operate and maintain shot blasting equipment

MSFFL2013 Select, operate and maintain demolition equipment

MSFFL2014 Select, operate and maintain scarifying equipment

# Performance Evidence

There must be evidence the candidate has completed the tasks outlined in the elements and performance criteria of this unit, and:

prepared 2 surfaces in accordance with finish requirements using powered floor preparation equipment, with at least one being a grinding preparation.

# Knowledge Evidence

There must be evidence the candidate has knowledge of:

how to identify subfloors where specialist preparation is required and the different types of preparation methods available to achieve the required surface quality

floor preparation using equipment:

organisational and site standards, requirements, policies and procedures

Australian Standards requirements

workplace safety system requirements

how to access manufacturer instructions

environmental protection and sustainability requirements

work flow for different types of floor preparation

types, characteristics, uses, and limitations of different floor preparation equipment, including how to operate equipment used for:

floor demolition

grinding – single headed, double-headed, planetary action (tooling selection requirements)

scarifying

shot blasting

sanding with rotary floor sander

fixtures and fittings that need to be removed prior to surface preparation

when wet method cleaning is required prior to mechanical methods being used

cleaning products suited to different types of contaminant and how they are used

processes for clean-up and disposal of waste slurry

procedures for the recording, reporting and maintenance of workplace records and information.

# Assessment Conditions

Skills must have been demonstrated in the workplace or in a simulated environment that reflects workplace conditions and contingencies. The following conditions must be met for this unit:

use of suitable facilities, equipment and resources, including:

floors requiring preparation

grinding equipment

surfaces to be prepared

work orders

workplace procedures.

Assessors must satisfy the NVR/AQTF mandatory competency requirements for assessors.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0601ab95-583a-4e93-b2d4-cfb27b03ed73