# Assessment Requirements for MSFFL2038 Install lay flat vinyl floor coverings

Release: 1

Assessment Requirements for MSFFL2038 Install lay flat vinyl floor coverings

# Modification History

Release 1. Supersedes and is equivalent to MSFFL2021 Install lay flat vinyl floor coverings.

# Performance Evidence

There must be evidence the candidate has completed the tasks outlined in the elements and performance criteria of this unit, and:

cut, laid and fixed flat vinyl floor coverings in accordance with job requirements:

to a single room with external corners, including pattern match at seams, using freehand, template or scribing method, with seams to be cold-welded

to connecting rooms, including pattern match at seams and cross joins using free-hand, template or scribing method, with seams to be cold-welded

pattern match repair by double cutting the corresponding pattern, with seams to be cold-welded.

# Knowledge Evidence

There must be evidence the candidate has knowledge of:

mathematical procedures for estimation and measurement, including calculation of area and estimation of material requirements

lay flat vinyl flooring installation:

Australian Standards requirements

workplace safety system requirements

how to access manufacturer instructions

environmental protection and sustainability requirements

work flow

characteristics and requirements of sub-floor preparation

types, characteristics, uses and limitations of:

lay flat vinyl floor coverings including encapsulated glass fibre and text backing products

lay flat vinyl floor covering installation adhesives including water-based acrylic, water-based contact and solvent-based contact

cold chemical seam welding techniques

types, characteristics, safe use, maintenance and limitations of tools and equipment

type, characteristics, uses and limitations of other materials used

techniques for set out, laying and cutting, the types of problems that may occur and how to avoid and respond

quality indicators for completed work

procedures for the recording, reporting and maintenance of workplace records and information.

# Assessment Conditions

Skills must have been demonstrated in the workplace or in a simulated environment that reflects workplace conditions and contingencies.

The following conditions must be met for this unit:

use of suitable facilities, equipment and resources, including:

locations requiring lay flat vinyl installations

lay flat vinyl flooring

equipment, tools and materials to complete tasks in the Performance Evidence, including safety and personal protective equipment

work orders

workplace procedures.

Assessors must satisfy the NVR/AQTF mandatory competency requirements for assessors.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0601ab95-583a-4e93-b2d4-cfb27b03ed73