# MSFFL3008 Install patterned carpet floor coverings

Release: 1

MSFFL3008 Install patterned carpet floor coverings

# Modification History

Release 1 - New unit of competency

# Application

This unit covers installing woven and tufted carpet floor coverings containing geometric and decorative patterns using adhesive-bonded and carpet gripper systems.

Licensing, legislative or certification requirements may apply to this unit and relevant state/territory and local government agencies should be consulted to determine any necessary certification or licensing for undertaking flooring work. Access to construction sites requires certification of general induction training specified by the National Code of Practice for Induction for Construction Work (ASCC 2007).

# Pre-requisite Unit

# Competency Field

# Unit Sector

Flooring technology

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe the performance needed to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Prepare for installation | 1.1 | Applicable work health and safety (WHS), legislative and organisational requirements relevant to install patterned carpet floor covering are verified and complied with |
|  |  | 1.2 | Instructions and/or plans are read and interpreted to identify the process required to complete work tasks |
|  |  | 1.3 | Resources required in terms of labour, equipment, materials, transport, site loading and hoisting, storage facilities, air conditioning, heating, lighting, power supply, security and time are assessed |
|  |  | 1.4 | Tools, equipment and materials are selected and checked prior to use to ensure that they are appropriate for the work, serviceable and in a safe condition |
|  |  | 1.5 | Sequence of work is planned, including the starting point, to ensure efficiency and quality of installation |
| 2 | Set out materials | 2.1 | Carpet to be installed is checked against work order |
|  |  | 2.2 | Patterned carpet floor coverings are acclimatised according to manufacturer recommendations |
|  |  | 2.3 | Type of perimeter finishing, joining materials, gripper strip, metal mouldings and cover strips are selected following specification or work order |
|  |  | 2.4 | Underlay, gripper strips and metal mouldings are inspected to confirm correct fitting |
|  |  | 2.5 | Work area is cleared of scraps or other contaminants |
|  |  | 2.6 | Work lines and starting point are established |
| 3 | Lay patterned floor coverings | 3.1 | Instructions and sequence for laying and fixing are followed |
|  |  | 3.2 | Carpet pieces are laid out and checked for size and direction of pile |
|  |  | 3.3 | Stretching equipment, joining and fixing methods are used following manufacturer instructions and Australian Standards |
|  |  | 3.4 | Cover and finishing strips are suitably fixed for the type of sub-floor |
|  |  | 3.5 | Protective coverings are installed as specified |
|  |  | 3.6 | Finished work is inspected for compliance with work order, workplace and Australian Standards of finish |
| 4 | Complete housekeeping | 4.1 | Unused materials are stored or recycled as required |
|  |  | 4.2 | Tools and equipment are cleaned and stored appropriately |
|  |  | 4.3 | Work area is cleaned and rubbish disposed of appropriately |
|  |  | 4.4 | Workplace documentation is completed |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency. Detail on appropriate performance levels for each furnishing unit of competency in reading, writing, oral communication and numeracy utilising the Australian Core Skills Framework (ACSF) are provided in the Furnishing Training Package Implementation Guide.

# Range of Conditions

| Specifies different work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) are included. Range is restricted to essential operating conditions and any other variables essential to the work environment. |  |
| --- | --- |
| Unit context includes: | WHS requirements, including legislation, building codes, material safety management systems, hazardous and dangerous goods codes, and local safe operating procedures or equivalent work is carried out in accordance with legislative obligations, environmental legislation, relevant health regulations, manual handling procedures and organisation insurance requirements work requires individual to demonstrate some discretion, judgement and problem solving |
| Patterned carpet floor coverings include: | tufted carpet woven carpet fusion-bonded carpet |
| Tools and equipment include: | carpet knee kicker power stretcher carpet stair tool hand trucks carpet trolleys hammer utility knife hook knife carpet wall trimmer heat seam iron spiked carpet roller weight or seam roller awl carpet spreader carpet shears napping shears loop pile cutter cushion backed cutter moulding lifter sewing needle |
| Materials include: | commercial architectural carpet gripper strips metal mouldings and PVC trims heat seam tape hessian tape thread adhesives |
| Personal protective equipment includes: | that prescribed under legislation, regulations and enterprise policies and practices |
| Information and procedures include: | workplace procedures relating to the use of tools and equipment work instructions, including job sheets, cutting lists, plans, drawings and designs workplace procedures relating to reporting and communication manufacturer specifications and operational procedures |

# Unit Mapping Information

Supersedes and is equivalent to LMFFL3204A Install patterned carpet floor coverings

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0601ab95-583a-4e93-b2d4-cfb27b03ed73

Assessment Requirements for MSFFL3008 Install patterned carpet floor coverings

# Modification History

Release 1 - New unit of competency

# Performance Evidence

Interpret work order and locate and apply relevant information

Apply safe handling requirements for equipment, products and materials, including use of personal protective equipment

Identify materials used in the work process

Follow work instructions, operating procedures and inspection processes to:

minimise the risk of injury to self or others

prevent damage to goods, equipment and products

maintain required production output and product quality

Install woven patterned carpet requiring pattern matching at seams and cross joints using the carpet gripper strip installation system

Install tufted patterned carpet requiring pattern matching at seams and cross joints using the carpet gripper strip installation system

Install tufted patterned carpet using adhesive bonded methods of installation including pattern matching at seams and cross joints

Use mathematical ideas and techniques to correctly complete measurements, calculate area and estimate material requirements

Communicate ideas and information to enable confirmation of work requirements and specifications and the reporting of work outcomes and problems, interpret basic plans and follow safety procedures

Avoid backtracking, work flow interruptions or wastage

Work with others and in a team by recognising dependencies and using cooperative approaches to optimise work flow and productivity

# Knowledge Evidence

State or territory WHS legislation, regulations, standards and codes of practice relevant to the installation of patterned carpet

Organisational and site standards, requirements, policies and procedures for the installation of patterned carpet

Environmental protection requirements

Established communication channels and protocols

Relevant problem identification and resolution

Types of tools and equipment and procedures for their safe use, operation and maintenance

Characteristics of materials, products and defects

Set-up and operation of equipment

Procedures for the recording, reporting and maintenance of workplace records and information

# Assessment Conditions

Assessors must:

hold training and assessment competencies as determined by the National Skills Standards Council (NSSC) or its successors

have vocational competency in the furnishing industry at least to the level being assessed with broad industry knowledge and experience, usually combined with a relevant industry qualification

be familiar with the current skills and knowledge used and have relevant, current experience in the furnishing industry.

Assessment methods must confirm consistency of performance over time rather than a single assessment event and in a range of workplace relevant contexts.

Assessment must be by observation of relevant tasks with questioning on underpinning knowledge and, where applicable, multimedia evidence, supervisor’s reports, projects and work samples.

Assessment is to be conducted on single units of competency or in conjunction with other related units of competency. Foundation skills are integral to competent performance in the unit and should not be assessed separately.

Assessment must occur on the job or in a workplace simulated facility with relevant process, equipment, materials, work instructions and deadlines.

Access is required to materials and equipment and specifications relevant to installing patterned carpet floor coverings

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0601ab95-583a-4e93-b2d4-cfb27b03ed73