# Assessment Requirements for MSFFL3067 Complete large scale and complex vinyl flooring installations

Release: 1

Assessment Requirements for MSFFL3067 Complete large scale and complex vinyl flooring installations

# Modification History

Release 1. Supersedes and is not equivalent to MSFFL3013 Install commercial vinyl floor coverings, and, MSFFL3021 Cut and install resilient floor coverings.

# Performance Evidence

There must be evidence the candidate has completed the tasks outlined in the elements and performance criteria of this unit, and:

performed at least 4 resilient floor installations – 2 using heterogeneous sheet vinyl and 2 using homogeneous sheet vinyl, that cover all of the following at least once, in any combination:

all of the following installation types:

cove installation

flat installation

stair installations with a metal stair nosing

all of the following characteristics:

pencil cove

cove fillet

border cove

internal and external mitred corners

more than two widths of vinyl

hallway or passage with connecting room

all of the following techniques:

trimming selvedges

bar scribing

direct scribing

template scribing

under scribing

pattern matching

heat weld joins.

# Knowledge Evidence

There must be evidence the candidate has knowledge of:

mathematical procedures for estimation and measurement, including calculation of area and estimation of material requirements

resilient floor covering installation:

Australian Standards requirements

environmental protection and sustainability requirements

workplace safety system requirements

how to access manufacturer instructions

work flow

types and characteristics of large commercial areas

types and characteristics of different types of stairs, including:

box steps

bullnose steps

circular

kit winder treads

open newell

open tread

recessed steps

types and characteristics, uses and limitations of different resilient floor coverings:

homogenous and heterogeneous

linoleum sheet

encapsulated glass fibre

rubber sheet

characteristics and requirements of subfloor preparation:

concrete

timber

types, characteristics, uses and limitations of different adhesives, including:

waterbased acrylic

two-part epoxy

two-part polyurethane

single pack polyurethane

water-based and solvent-based contact adhesive

types, characteristics, safe use, maintenance and limitations of tools, equipment and materials

techniques for set out, laying, cutting, welding and fixing, the types of problems that may occur and how to avoid and respond, including:

cold chemical welding

heat welding

scribing – direct and template

seam preparation

quality indicators for completed work

procedures for the recording, reporting and maintenance of workplace records and information.

# Assessment Conditions

Skills must have been demonstrated in the workplace or in a simulated environment that reflects workplace conditions and contingencies.

The following conditions must be met for this unit:

use of suitable facilities, equipment and resources, including:

areas to be covered as per Performance Evidence

resilient floor coverings

equipment, tools and materials to complete tasks in the Performance Evidence, including safety and personal protective equipment

work orders

workplace procedures.

Assessors must satisfy the NVR/AQTF mandatory competency requirements for assessors.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0601ab95-583a-4e93-b2d4-cfb27b03ed73