# MSFFL3107 Assess flooring installation sites

Release: 1

MSFFL3107 Assess flooring installation sites

# Modification History

Release 1. Supersedes and is equivalent to MSFFL3054 Assess flooring installation sites. Changes to application, elements and performance criteria to improve clarity and consistency across MSFFL units. Changes to assessment requirements.

# Application

This unit describes the skills and knowledge required to inspect and assess flooring installation sites and report with recommendations about the condition of timber and concrete subfloor structures and subfloors and about factors that may affect the installation of flooring coverings including underlays on them.

Workers with this competency undertake routine tasks on residential and non-residential/commercial sites according to job instructions and workplace procedures using discretion, judgement and problem-solving to identify and resolve routine issues. They work with a high degree of self-direction under routine supervision, and they are responsible for their output.

The work described in this unit may require a licence or certification in some jurisdictions, and state or territory authorities should be consulted to determine the current status. Workers on construction sites are required by law to complete induction training. They must also comply with relevant legislation, regulations, Australian Standards, codes and manufacturer installation instructions.

# Competency Field

Flooring technology

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Prepare to assess flooring installation sites | 1.1 Identify from the customer’s enquiry or job instructions the type of the proposed floor coverings and other relevant work requirements 1.2 Identify hazards relating to the work site, work practices, materials, tools and equipment, and check risk treatments are in place and operating 1.3 Select and check all required inspection, testing and other tools and equipment are ready to use, and organise to fix any deficiencies according to workplace procedures 1.4 Determine the subfloor surface requirements of the proposed floor coverings from manufacturer instructions |
| 2. Assess flooring installation environments | 2.1 Identify aspects of the internal and external environment that may affect the installation of floor coverings 2.2 Investigate the potential interactions between the environment and the proposed floor coverings |
| 3. Assess subfloor structures and subfloors | 3.1 Visually inspect the subfloor structure and subfloor to identify the materials used 3.2 Run and check datum lines and confirm dimensions are accurate 3.3 Complete tests appropriate to the type of floor covering to be installed according to manufacturer instructions |
| 4. Report findings according to workplace procedures | 4.1 Identify remedies to any faults identified and make recommendations for floor coverings 4.2 Prepare an assessment report 4.2 Make copies of the report available to appropriate people |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential to performance but not explicit in the performance criteria.

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Unit Mapping Information

Supersedes and is equivalent to MSFFL3054 Assess flooring installation sites.

# Links

Companion Volume Implementation Guides are found in VETNet – - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0601ab95-583a-4e93-b2d4-cfb27b03ed73

Assessment Requirements for MSFFL3107 Assess flooring installation sites

# Modification History

Release 1. Supersedes and is equivalent to MSFFL3054 Assess flooring installation sites. Changes to application, elements and performance criteria to improve clarity and consistency across MSFFL units. Changes to assessment requirements.

# Performance Evidence

There must be evidence the candidate has completed the tasks in the elements and performance criteria of this unit of competency and has demonstrated the ability to complete and prepare a report about the findings of a full internal and external site assessment according to job instructions for:

one timber subfloor structure and subfloor

one concrete subfloor.

# Knowledge Evidence

There must be evidence the candidate has knowledge of:

pre-installation inspections:

how subfloors must be prepared for carpet and underlay

the safe and correct use of and maintenance requirements for tools and equipment used for inspections

type, characteristics and common defects of timber and concrete subfloor structures, including the role of moisture and how it affects the installation of flooring materials

how to conduct, according to Australian standards, the following moisture tests on timber and concrete subfloors:

a visual inspection for signs of moisture

an electrical resistance moisture test on timber flooring, floor joists, bearers and stumps

a calcium chloride test on concrete flooring

a hygrometer (in-situ) moisture test on concrete flooring

a capacitance moisture test on concrete flooring

how to conduct a flatness and falls assessment

inspection items required for timber flooring:

datum line running and checking (relating to floor fall, dips, irregularities and finished floor height in relation to other floor coverings and door clearances)

protruding nails

loose metal and other objects

cracked, loose, rotting or damaged boards

insect attack

cross-flow ventilation

inspection items required for concrete flooring:

datum line running and checking

protruding obstacles

cracks or damage (identifying static versus dynamic)

damage to expansion joints

concrete surface for weakness

the presence of concrete sealers and/or surface treatments

existing floor coatings

oil, fats, grease, and/or detergent contamination

transitions to other flooring systems

drainage requirements including falls to drains

ramps

aspects of the internal and external environment that must be inspected in addition to the subfloor

formats and required inclusions for assessment reports, including the:

type of inspection completed

testing methods used

inspection’s findings

suggested remedies to any faults identified

recommendations for floor coverings or finishing

workplace policies and procedures relevant to their job role and responsibilities including for:

conducting assessments

communication and reporting including reporting findings

quality indicators for completed work and process improvements

work health and safety, including the use of personal protective equipment and safety data sheets

work planning.

# Assessment Conditions

Skills must have been demonstrated in the workplace or in a simulated environment that reflects workplace conditions and contingencies. The following conditions must be met for this unit:

use of suitable facilities, equipment and resources, including:

installation sites for flooring materials including subfloors and their supporting structures

equipment, tools and materials required for the activities set out in the elements and performance criteria, including those to control hazards

workplace procedures.

Assessors must satisfy the NVR/AQTF mandatory competency requirements for assessors.

# Links

Companion Volume Implementation Guides are found in VETNet – - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0601ab95-583a-4e93-b2d4-cfb27b03ed73