# MSFFL3111 Install commercial carpet

Release: 1

MSFFL3111 Install commercial carpet

# Modification History

Release 1. Supersedes and is equivalent to MSFFL3061 Install commercial carpet. Changes to application, elements and performance criteria to improve clarity and consistency across MSFFL units. Changes to assessment requirements.

# Application

This unit describes the skills and knowledge required to install commercial-grade carpet to multiple connecting rooms, large commercial areas and in multi-storey buildings. It does not include installing:

underlay and grippers (which MSFFL3103 Install soft carpet underlay and grippers addresses)

carpet using the direct-stick or dual-bond methods (which MSFFL3110 Install adhesive-fixed carpet addresses)

carpet on complex stairs (which MSFFL3113 Install carpet on complex stairs addresses).

Workers with this competency undertake routine tasks on residential and non-residential/commercial sites according to job instructions including plans and specifications and workplace procedures using discretion, judgement and problem-solving to identify and resolve routine issues. They work with a high degree of self-direction under routine supervision, and they are responsible for their output.

The work described in this unit may require a licence or certification in some jurisdictions, and state or territory authorities should be consulted to determine the current status. Workers on construction sites are required by law to complete induction training. They must also comply with relevant legislation, regulations, Australian Standards, codes and manufacturer installation instructions.

# Competency Field

Flooring technology

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Prepare to install commercial carpet | 1.1 Interpret carpet installation requirements from job and manufacturer instructions and calculate the quantities of materials required 1.2 Identify hazards relating to the work site, work practices, materials, tools and equipment, and check risk treatments are in place and operating 1.3 Select and check all required tools and equipment are ready to use, and organise to fix any deficiencies according to workplace procedures 1.4 Gather the carpet and finishing, joining, fastening and other materials required by job instructions, and check they meet quality requirements and are prepared and ready to install according to manufacturer instructions 1.5 Confirm the subfloor, underlay and grippers are clean and suitable for installing the carpet, and fix or report any deficiencies according to workplace procedures |
| 2. Cut, fit and fasten the commercial carpet | 2.1 Determine the set out and laying direction of the carpet according to job instructions to minimise waste and joins, consider the direction of the pile, and ensure an efficient, quality installation 2.2 Establish the skirting point and seam placements for the carpet 2.3 Lay out the carpet pieces and check them for size, direction of pile and pattern match 2.4 Measure, mark, cut, seal and join the carpet pieces according to workplace procedures 2.5 Position, stretch and fasten the carpet according to the manufacturer instructions 2.6 Trim off excess carpet, install finishing strips, and finish installing the carpet to meet quality requirements 2.7 Check the carpet is installed correctly according to job instructions and quality requirements, and fix any deficiencies |
| 3. Complete the task according to workplace procedures | 3.1 Clean up the work area, identify items for recycling and dispose of waste 3.2 Identify, tag and report any damage to equipment and tools 3.3 Check, clean and do routine maintenance on equipment and tools and store them and unused materials 3.4 Complete task outcome documentation |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential to performance but not explicit in the performance criteria.

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Unit Mapping Information

Supersedes and is equivalent to MSFFL3061 Install commercial carpet.

# Links

Companion Volume implementation guides are found in VETNet – - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0601ab95-583a-4e93-b2d4-cfb27b03ed73

Assessment Requirements for MSFFL3111 Install commercial carpet

# Modification History

Release 1. Supersedes and is equivalent to MSFFL3061 Install commercial carpet. Changes to application, elements and performance criteria to improve clarity and consistency across MSFFL units. Changes to assessment requirements.

# Performance Evidence

There must be evidence the candidate has completed the tasks in the elements and performance criteria of this unit of competency and has demonstrated the ability to complete according to job instructions:

at least three commercial carpet installations — where the subfloor has been adequately prepared — that include every one of the following at least once, in any combination:

carpet:

commercial woven carpet

commercial tufted carpet

areas:

an area containing at least two rooms connected by a common corridor

an area containing at least seven lineal metres of carpet in length that must be split and joined

an area with an obstacle that requires the carpet to be stretched up to it (such as a lift lobby, reception area or column)

three seam joins and three cross joins

a box step stairway with at least three steps including the installation of commercial-grade stair nosing

a kite winder treads stairway with at least three steps

a bullnose step.

# Knowledge Evidence

There must be evidence the candidate has knowledge of:

manufacturer installation instructions and the relevant Australian standards for commercial carpet

the safe and correct use of and maintenance requirements for tools and equipment including power stretchers for installing commercial carpet

the safe and correct handling of heavy carpet loads

the most common issues that can arise when installing commercial carpet and how to avoid and respond to them

workplace policies and procedures relevant to their job role and responsibilities including for:

installing commercial carpet

communication and reporting

environmental protection and sustainability, including maximising resource utilisation and minimising waste

quality indicators for completed work and process improvements

work health and safety, including the use of personal protective equipment and safety data sheets

work planning

how to identify if a subfloor is suitable for installing commercial carpet

how to identify if carpet underlay, grippers and commercial carpet are installed correctly.

# Assessment Conditions

Skills must have been demonstrated in the workplace or in a simulated environment that reflects workplace conditions and contingencies. The following conditions must be met for this unit:

use of suitable facilities, equipment and resources, including:

locations in which to install commercial carpet and which already have underlay and grippers installed

commercial carpet

equipment and tools required for the activities set out in the elements and performance criteria, including those to control hazards

job instructions and workplace procedures.

Assessors must satisfy the NVR/AQTF mandatory competency requirements for assessors.

# Links

Companion Volume implementation guides are found in VETNet – - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0601ab95-583a-4e93-b2d4-cfb27b03ed73