# MSFFL3127 Install synthetic textile floor coverings in indoor and outdoor facilities

Release: 1

MSFFL3127 Install synthetic textile floor coverings in indoor and outdoor facilities

# Modification History

Release 1. Supersedes and is equivalent to MSFFL3063 Install synthetic textile floor coverings to sports facilities and outdoor spaces. Changes to title, application, elements and performance criteria to improve clarity and consistency across MSFFL units. Changes to assessment requirements.

# Application

This unit describes the skills and knowledge required to install synthetic turf and needle-punch floor coverings in indoor and outdoor facilities, including the preparation, cutting, laying out, joining, fixing and finishing of the synthetic textile floor covering material.

Workers with this competency undertake routine tasks on residential and non-residential/commercial sites according to job instructions and workplace procedures using discretion, judgement and problem-solving to identify and resolve routine issues. They work with a high degree of self-direction under routine supervision, and they are responsible for their output.

The work described in this unit requires a licence or certification in some jurisdictions, and state or territory authorities should be consulted to determine the current status. Workers on construction sites are required by law to complete induction training. They must also comply with Australian Standards, codes and manufacturer installation instructions.

# Competency Field

Flooring technology

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Prepare to install synthetic textile floor coverings | 1.1 Interpret floor covering installation requirements from job and manufacturer instructions and calculate the quantities of materials required 1.2 Identify hazards relating to the work site, work practices, materials, tools and equipment, and check risk treatments are in place and operating 1.3 Select and check all required tools and equipment are ready to use, and organise to fix any deficiencies according to workplace procedures 1.4 Gather the floor coverings and other materials required by job instructions, and check they meet quality requirements and are prepared and ready to install according to manufacturer instructions 1.5 Confirm the subfloor is clean and suitable for installing the synthetic floor covering material, and fix or report any deficiencies according to workplace procedures |
| 2. Cut, fit and fasten synthetic textile floor coverings | 2.1 Cut and lay required shock padding/underlay for the floor covering according to job instructions, manufacturer instructions and workplace procedures 2.2 Determine the set out and laying direction of the floor covering according to job instructions to minimise waste and joins, consider the direction of the pile and ensure an efficient, quality installation 2.3 Establish the skirting point and seam placements for the floor covering 2.4 Lay out the synthetic floor covering material and check it for size, direction of pile and pattern match 2.5 Measure, mark, cut, seal and join the synthetic floor covering material according to workplace procedures and manufacturer instructions 2.6 Position, stretch and fasten the synthetic floor covering according to workplace procedures 2.7 Fit the synthetic floor covering material to finishing strips, trim excess material to walls and finish installing the material according to workplace procedures 2.8 Check the synthetic floor covering material is installed correctly according to job instructions and quality requirements, and fix any deficiencies 2.9 Check the floor covering is installed correctly according to job instructions and quality requirements, and fix any deficiencies |
| 3. Complete the task according to workplace procedures | 3.1 Clean up the work area, identify items for recycling and dispose of waste 3.2 Identify, tag and report any damage to equipment and tools 3.3 Check, clean and perform routine maintenance on equipment and tools and store them and unused materials 3.4 Complete task outcome documentation |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential to performance but not explicit in the performance criteria.

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Unit Mapping Information

Supersedes and is equivalent to MSFFL3063 Install synthetic textile floor coverings to sports facilities and outdoor spaces.

# Links

Companion Volume Implementation Guides are found in VETNet – - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0601ab95-583a-4e93-b2d4-cfb27b03ed73

Assessment Requirements for MSFFL3127 Install synthetic textile floor coverings in indoor and outdoor facilities

# Modification History

Release 1. Supersedes and is equivalent to MSFFL3063 Install synthetic textile floor coverings to sports facilities and outdoor spaces. Changes to title, application, elements and performance criteria to improve clarity and consistency across MSFFL units. Changes to assessment requirements.

# Performance Evidence

There must be evidence the candidate has completed the tasks in the elements and performance criteria of this unit of competency and has demonstrated the ability to complete according to job instructions:

at least two installations of synthetic textile floor coverings that also require the preparation of the subfloor including:

one synthetic turf grass to an outdoor installation or recreational or sports area

one synthetic needle-punch to an indoor installation or recreational or sports area

installation of synthetic textile floor coverings — where the stairs have been adequately prepared — on a box step stairway with at least three steps, including the fitting of appropriate stair nosing.

# Knowledge Evidence

There must be evidence the candidate has knowledge of:

manufacturer installation and use instructions for:

bonded foam, latex foam, rubber and styrene-butadiene rubber (SBR) underlay and/or shock pads

synthetic textile floor coverings, including needle punch carpet, PVC-backed carpet, rubber-backed carpet and unfilled, sand-filled and rubber-filled synthetic turf

adhesives, including plasticised and rubber resin acrylic adhesive, solvent-based outdoor / grass adhesive, two-part polyurethane and water, solvent-based and solvent-free adhesives

the safe and correct use of, and maintenance requirements for tools and equipment for, installing synthetic textile floor covering

the safe and correct handling of heavy synthetic textile floor covering loads

the most common issues that can arise when installing synthetic textile floor coverings and how to avoid and respond to them

workplace policies and procedures relevant to their job role and responsibilities including for:

installing synthetic textile floor coverings

communication and reporting

environmental protection and sustainability, including maximising resource utilisation and minimising waste

quality indicators for completed work and process improvements

work health and safety, including the use of personal protective equipment and safety data sheets

work planning

how to identify if a subfloor is suitable for installing synthetic textile floor coverings

how to identify if synthetic textile floor coverings are installed correctly.

# Assessment Conditions

Skills must have been demonstrated in the workplace or in a simulated environment that reflects workplace conditions and contingencies. The following conditions must be met for this unit:

use of suitable facilities, equipment and resources, including:

locations in which to install synthetic textile floor coverings

synthetic floor covering

equipment and tools required for the activities set out in the elements and performance criteria, including those to control hazards

job instructions and workplace procedures.

# Links

Companion Volume Implementation Guides are found in VETNet – - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0601ab95-583a-4e93-b2d4-cfb27b03ed73