# MSFFM2016 Join solid timber

Release: 1

MSFFM2016 Join solid timber

# Modification History

Release 1. Changes to code, application, elements, performance criteria, assessment requirements. Foundation skills added. Range of conditions removed. Assessment requirements changed.

# Application

This unit describes the skills and knowledge required to join timber to manufacture solid timber flat surfaces.

Workers with this competency follow established procedures in timber technology industries such as cabinet and furniture making using discretion, judgement and some problem solving.

No licensing or certification requirements exist at the time of publication. Relevant legislation, industry standards and codes of practice within Australia must be applied.

# Competency Field

Cabinetry and furniture

# Elements and Performance Criteria

| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| --- | --- |
| 1 Prepare for task | 1.1 Identify task requirements from work documentation 1.2 Plan work sequence and identify procedures for checking quality at each stage 1.3 Identify and follow work health and safety procedures 1.4 Select suitable work area for task 1.5 Select tools and equipment to meet task requirements and check that they are safe and functional |
| 2 Lay out and prepare materials | 2.1 Obtain timbers and check they meet task quality requirements 2.2 Identify joining process to meet task requirements 2.3 Prepare and lay out timbers to facilitate efficient joining process |
| 3 Join timber | 3.1 Accurately measure, mark and cut timbers to size 3.2 Use tools and equipment safely and in accordance with manufacturer specifications 3.3 Apply adhesives in accordance with workplace procedures and manufacturer instructions 3.4 Complete final trim and finishing to meet task specifications 3.5 Check that joint meets required quality standards and rectify any deficiencies |
| 4 Finalise task | 4.1 Clean work area, safely dispose of waste and identify items for recycling in accordance with workplace procedures and environmental standards 4.2 Clean machinery, complete routine maintenance and leave in a safe mode in accordance with workplace procedures 4.3 Identify, tag and report faulty equipment in accordance with workplace procedures 4.4 Complete task documentation in accordance with workplace procedures |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential to performance but not explicit in the performance criteria.

Reading skills to interpret familiar workplace procedures and work documentation

Writing skills to communicate simple factual information in familiar workplace documents

Numeracy skills to measure and calculate lengths and areas and material requirements

Other foundation skills essential to performance are explicit in the performance criteria of this unit.

# Unit Mapping Information

Release 1. Supersedes and is equivalent to MSFFM2005 Join solid timber.

# Links

Companion Volume implementation guides are found in VETNet – - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0601ab95-583a-4e93-b2d4-cfb27b03ed73

Assessment Requirements for MSFFM2016 Join solid timber

# Modification History

Release 1. Changes to code, application, elements, performance criteria, assessment requirements. Foundation skills added. Range of conditions removed. Assessment requirements changed.

# Performance Evidence

There must be evidence the candidate has completed the tasks outlined in the elements and performance criteria of this unit, and demonstrated the ability to:

join at least 4 solid timber tops, shelves or panels, each with a minimum of 3 sections

use each of the following edge joining methods at least once to achieve workplace quality standards:

butt and dowel

moulded edge joint (finger-joint, tongue in groove, or zigzag)

biscuit joint or domino

slip-tongue (aka tongue-in-groove) joint.

# Knowledge Evidence

There must be evidence the candidate has knowledge of:

workplace procedures for:

work health and safety

quality assurance

reporting of machinery faults and material defects

the following wood characteristics and defects:

bowing

cupping

expansion

grain direction

spring

twisting

characteristics of different types of adhesives used to join solid timber and their application methods

types of sawing methods, including back sawn and quarter sawn, and the impact of these on joining

tools, materials, equipment, personal protective equipment (PPE) and processes used in timber joining.

# Assessment Conditions

Skills must have been demonstrated in the workplace or in a simulated environment that reflects workplace conditions and contingencies. The following conditions must be met for this unit:

use of suitable facilities, equipment and resources, including:

work documentation

tools, equipment and machinery required for performance evidence

PPE

timber materials for joining

workplace procedures.

# Links

Companion Volume implementation guides are found in VETNet – - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0601ab95-583a-4e93-b2d4-cfb27b03ed73