# Assessment Requirements for MSFFM3038 Set up, operate and maintain automated edge banding machines

Release: 1

Assessment Requirements for MSFFM3038 Set up, operate and maintain automated edge banding machines

# Modification History

Release 1. Changes to code, application, elements, performance criteria, assessment requirements. Foundation skills added. Range of conditions removed. Assessment requirements changed.

# Performance Evidence

There must be evidence the candidate has completed the tasks outlined in the elements and performance criteria of this unit, and demonstrated the ability to:

set up and operate an automated (not fully automatic) edging machine to apply at least 3 different edge treatment materials

complete operator maintenance at least once, including:

checking that guards, safety switches, and machine jigs and attachments are functioning correctly

servicing machine lubrication points

identifying tooling needing sharpening and repair

checking that drive belts are undamaged and correctly tensioned

cleaning machine at the end of a production run

checking that electrical systems are functioning, and identifying issues for referral to qualified electrician

completing maintenance documentation.

# Knowledge Evidence

There must be evidence the candidate has knowledge of:

workplace procedures for:

work health and safety

quality assurance and tolerance levels

reporting of machinery faults and material defects

functions and characteristics of edge banding machines:

pre-milling technology

standard automated edgers (not fully automatic)

soft forming machines

risk control measures, including personal protective equipment (PPE), for the following hazards

cutting and crushing hazard

objects and particles thrown by machine

noise

airborne contaminants

slips, trips and falls

manual handling injuries

characteristics of timber and board products and adhesives commonly used with edge banding machines, and of these edging products:

solid timber

laminates

plastics

non-ferrous materials and foils.

# Assessment Conditions

Skills must have been demonstrated in the workplace or in a simulated environment that reflects workplace conditions and contingencies. The following conditions must be met for this unit:

use of suitable facilities, equipment and resources, including:

work documentation

automated edge banding machines required for performance evidence

PPE

unprocessed materials for edge banding

workplace procedures.

Assessors must satisfy the NVR/AQTF mandatory competency requirements for assessors.

# Links

Companion Volume implementation guides are found in VETNet – - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0601ab95-583a-4e93-b2d4-cfb27b03ed73