# MSFGG2010 Fabricate and install insect and security screens

Release: 1

MSFGG2010 Fabricate and install insect and security screens

# Modification History

Release 1 - New unit of competency

# Application

This unit of competency covers measuring, fabricating and installing insect screens, heavy-duty security mesh, security doors and window grills using aluminium.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

# Competency Field

# Unit Sector

Glass and Glazing

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe the performance needed to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Plan for work | 1.1 | Work requirements in the form of type of insect or security screen to be fabricated and installed are determined from work instructions, including the identification of its pattern or plan |
|  |  | 1.2 | Work health and safety (WHS) requirements for fabrication and installation of insect or security screens, including personal protective equipment, are observed throughout the work |
|  |  | 1.3 | The suitable process for fabricating and installing insect or security screens is identified |
|  |  | 1.4 | Quantities of materials required are calculated from plans and/or work instructions and/or by measuring the screen opening in accordance with Australian Standards and industry practice |
|  |  | 1.5 | Cutting lists are prepared from plans, work instructions or measurements of opening to suit the type of screen to be installed |
|  |  | 1.6 | Fittings, fixings and joining devices or materials are selected in accordance with specifications or type of materials to be joined or fixed |
|  |  | 1.7 | Tools and equipment appropriate to the work method are identified and assembled for use |
| 2 | Prepare for work | 2.1 | Work sequence is determined in a logical order to suit the job and workplace procedures |
|  |  | 2.2 | Tools, equipment and materials are selected and checked prior to use to ensure they are appropriate for the work, serviceable and in a safe condition |
|  |  | 2.3 | Components of the insect or security screen are checked against specifications or job order for size, quantity, finish and profile |
| 3 | Fabricate and install insect or security screen | 3.1 | Materials are measured and marked to cutting list requirements and most economical use of materials |
|  |  | 3.2 | Materials are cut and stamped to cutting list requirements in accordance with workplace procedures or industry practice |
|  |  | 3.3 | Components of insect or security screen are assembled in accordance with recognised industry standards |
|  |  | 3.4 | Fittings are installed to screen in accordance with workplace procedures or industry practice |
|  |  | 3.5 | Insect or security screen is installed in opening in accordance with recognised industry standards |
| 4 | Complete work | 4.1 | Installed insect or security screen is inspected for quality of work and repaired, re-fabricated or reinstalled, as required, in accordance with workplace procedures |
|  |  | 4.2 | Material which can be reused is collected and stored |
|  |  | 4.3 | Waste and scrap materials are removed for disposal or recycling, as required |
|  |  | 4.4 | Work area is cleaned and rubbish disposed of, as appropriate |
|  |  | 4.5 | Tools, equipment and unused materials are cleaned and removed and stored appropriately |
|  |  | 4.6 | Workplace documentation is completed in accordance with workplace requirements |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency. Detail on appropriate performance levels for each furnishing unit of competency in reading, writing, oral communication and numeracy utilising the Australian Core Skills Framework (ACSF) are provided in the Furnishing Training Package Implementation Guide.

# Range of Conditions

| Specifies different work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) are included. Range is restricted to essential operating conditions and any other variables essential to the work environment. |  |
| --- | --- |
| Unit context includes: | WHS requirements, including legislation, building codes, material safety management systems, hazardous and dangerous goods codes, and local safe operating procedures or equivalent work is carried out in accordance with legislative obligations, environmental legislation, relevant health regulations, manual handling procedures and organisation insurance requirements work requires individuals to demonstrate some discretion, judgement and problem solving |
| Insect and security screens include: | fixed, sliding and emergency exit grills to suit various commercial and domestic window types sliding and hinged door applications |
| Materials include: | extruded door framing corner stakes interlocks fly mesh heavy-duty security mesh fittings and fixing/fasteners (screws and pop rivets) anti-jemmy protection hardware |
| Fittings include: | hinges wheels lock mechanisms that comply with Australian Standards door closures lock guards |
| Tools and equipment include: | assembly benches drop and up-cut saws pneumatic and manual presses stamping and notching tools copy routers end millers crimpers electric and pneumatic powered drills and screwdrivers pop rivet guns fly screen spline rollers soft rubber mallets knives bolt cutters or mechanical shears |
| Personal protective equipment includes: | that prescribed under legislation, regulation and enterprise policies and practices: gauntlets gloves safety glasses hard hats safety footwear aprons and overalls |
| Information and procedures include: | workplace procedures relating to the use of tools and equipment and personal protective equipment work instructions, including job sheets, cutting lists, plans, drawings and designs workplace procedures relating to reporting and communication manufacturer specifications and operational procedures |

# Unit Mapping Information

Supersedes and is equivalent to LMFGG2010C Fabricate and install insect and security screens.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0601ab95-583a-4e93-b2d4-cfb27b03ed73

Assessment Requirements for MSFGG2010 Fabricate and install insect and security screens

# Modification History

Release 1 - New unit of competency

# Performance Evidence

Interpret work order and locate and apply relevant information

Apply safe handling requirements for equipment, products and materials, including use of personal protective equipment

Identify materials used in the work process

Follow work instructions, operating procedures and inspection processes to:

minimise the risk of injury to self or others

prevent damage to goods, equipment and products

maintain required production output and product quality

Measure for security windows and doors and identify and select product materials to be used

Fabricate frame and components, including optimisation of materials, cutting, tooling, drilling and/or crimping to ensure compliance to Australian Standards

Select and install component hardware, including wheels, hinges, guides and locks

Assemble frames and/or grills to comply with Australian Standard requirements

Use mathematical ideas and techniques to correctly complete measurements, calculate area and estimate material requirements

Communicate ideas and information to enable confirmation of work requirements and specifications and the reporting of work outcomes and problems, interpret basic plans and follow safety procedures

Avoid backtracking, work flow interruptions or wastage

Work with others and in a team by recognising dependencies and using cooperative approaches to optimise work flow and productivity

# Knowledge Evidence

Work flow requirements in relation to the fabrication and installation of insect and security screens

Characteristics of materials used and uses of products produced

Workplace safety system requirements related to fabricating and installing insect and security screens

Correct identification of equipment, processes and procedures

Relevant Australian Standards

# Assessment Conditions

Assessors must:

hold training and assessment competencies as determined by the National Skills Standards Council (NSSC) or its successors

have vocational competency in the furnishing industry at least to the level being assessed with broad industry knowledge and experience, usually combined with a relevant industry qualification

be familiar with the current skills and knowledge used and have relevant, current experience in the furnishing industry.

Assessment methods must confirm consistency of performance over time rather than a single assessment event and in a range of workplace relevant contexts.

Assessment must be by observation of relevant tasks with questioning on underpinning knowledge and, where applicable, multimedia evidence, supervisor’s reports, projects and work samples.

Assessment is to be conducted on single units of competency or in conjunction with other related units of competency. Foundation skills are integral to competent performance in the unit and should not be assessed separately.

Assessment must occur on the job or in a workplace simulated facility with relevant process, equipment, materials, work instructions and deadlines.

Access is required to aluminium or other framing material, aluminium or other sheeting, fittings, fixing and fasteners, workplace operating procedures, personal protective equipment and an appropriate work area.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0601ab95-583a-4e93-b2d4-cfb27b03ed73