# Assessment Requirements for MSFGG2011 Glaze and reglaze residential windows and doors

Release: 1

Assessment Requirements for MSFGG2011 Glaze and reglaze residential windows and doors

# Modification History

Release 1. Supersedes and is equivalent to MSFGG2008 Glaze and re-glaze residential windows and doors.

# Performance Evidence

There must be evidence the candidate has completed the tasks outlined in the elements and performance criteria of this unit, and:

performed each of the following tasks at least once:

cut glass in accordance with Australian Standard AS1288

calculated glazing clearance and cutting size

hacked out and/or removed beads and glass safely from timber windows and doors

prepared glazing rebates and channels prior to installation

removed and disassembled aluminium window sashes and doors and removed glass safely

installed and/or adjusted unique, spiral or spring balances on double hung windows

taken template and glazed irregular shaped openings

glazed or reglazed each of the following at least once in accordance with AS1288:

timber putty faced window sashes or door panel

timber beaded:

sidelight

fixed panel

pivot or hinged or bi-fold doors

aluminium framed channel glazed door or window

aluminium framed beaded door or window with wedge rubbers

louvre blades.

# Knowledge Evidence

There must be evidence the candidate has knowledge of:

relevant Australian Standards, their purpose and key provisions:

AS 1288: Glass in buildings - Selection and installation

AS/NZS 2208: Safety glazing materials in buildings

AS/NZS 4667: Quality requirements for cut-to-size and processed glass

organisational customer communication standards and protocols

workplace safety system requirements related to the glazing of window and doorframes

work flow requirements in relation to fitting glass

qualities and characteristics of glass, including hazards and handling requirements for:

annealed glass

coated glass and insulated glass units

laminated glass patterned glass

tinted and heat reflective glass

toughened glass

wired glass

qualities and characteristics of different frame types and how this impacts the glazing process, including:

plastic

aluminium

steel

solid timber

factors that affect the fixing of glass and how this impacts on work, including:

compliance

specification

type of glass and frame

security requirements for specific locations

sealing methods

types, characteristics, safe use, maintenance and limitations of tools, equipment and materials, including:

glass handling and lifting equipment

hack-out knives

nail punches

pop rivet guns

pry bars

putty knives

rubber mallets

sealant guns

spiral balancer

sprig and/or diamond guns

tape measures

tungsten wheel glass cutters

vinyl cutters and rollers

types, characteristics, safe use and limitations of other materials, including:

adhesives

gaskets

glazing tapes

sealants

setting blocks

factors for consideration for products that include sash cords

glazing techniques for window and door frames, the types of problems that may occur and how to avoid and respond, including:

channel glazing

beaded glazing

double glazing

heal and toe glazing

quality indicators for glazed windows and doors

documentation requirements for completed glazing work.

# Assessment Conditions

Skills must have been demonstrated in the workplace or in a simulated environment that reflects workplace conditions and contingencies. The following conditions must be met for this unit:

use of suitable facilities, equipment and resources, including:

locations and/or frames requiring glazing and reglazing

frames and glass as specified in the Performance Evidence

equipment, tools and materials to perform tasks specified in Performance Evidence

work orders

workplace procedures.

Assessors must satisfy the NVR/AQTF mandatory competency requirements for assessors.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0601ab95-583a-4e93-b2d4-cfb27b03ed73