# MSFPF2005 Cut mat boards

Release: 1

MSFPF2005 Cut mat boards

# Modification History

Release 1 - New unit of competency

# Application

This unit of competency covers cutting out simple mat boards by hand and manual or computerised cutting machines for the mounting of artwork.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

# Competency Field

# Unit Sector

Picture Framing

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe the performance needed to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Prepare for cutting | 1.1 | Patterns, dimensions, materials and quantities are identified from work/job instructions |
|  |  | 1.2 | Work health safety (WHS) requirements relevant to the operation of cutting basic mat boards, including personal protection needs, are observed throughout the work |
|  |  | 1.3 | The process for obtaining materials/components and moving completed work to the next process is identified |
|  |  | 1.4 | Work area, tools, equipment and materials are selected and checked prior to use to ensure that they are appropriate for the work in terms of quality, serviceability and safety |
|  |  | 1.5 | Appropriate mat cutting machines are selected for the required cut and suitable blades are selected, fitted and adjusted for angled, circular or oval cuts |
|  |  | 1.6 | Cutting equipment components, controls and clamping systems are set |
|  |  | 1.7 | Board material is selected following work order or specification and laid out to meet requirements and minimise waste |
|  |  | 1.8 | Work sequence is planned to suit the job |
| 2 | Conduct cutting operation | 2.1 | Mat board is located on machine and clamped |
|  |  | 2.2 | Equipment is operated and monitored to ensure correct cut quality, output and that wastage is within allowable limits |
|  |  | 2.3 | Problems occurring during work operations are identified and reported to appropriate persons |
|  |  | 2.4 | Routine equipment adjustments are completed |
|  |  | 2.5 | Items which do not meet quality requirements are repaired or discarded |
|  |  | 2.6 | Finished work is inspected for compliance against specification or work order, workplace requirements and finish |
| 3 | Complete work | 3.1 | Mat boards are marked for identification and stored following workplace procedures |
|  |  | 3.2 | Scraps and off-cuts are removed for disposal/recycling as required by workplace procedures |
|  |  | 3.3 | Work area is cleaned and rubbish disposed of as appropriate |
|  |  | 3.4 | Workplace documentation is completed as required |
|  |  | 3.5 | Tools, equipment and unused materials are cleaned, stored and stock levels reported following workplace procedures |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency. Detail on appropriate performance levels for each furnishing unit of competency in reading, writing, oral communication and numeracy utilising the Australian Core Skills Framework (ACSF) are provided in the Furnishing Training Package Implementation Guide.

# Range of Conditions

| Specifies different work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) are included. Range is restricted to essential operating conditions and any other variables essential to the work environment. |  |
| --- | --- |
| Unit context includes: | WHS requirements, including legislation, building codes, material safety management systems, hazardous and dangerous goods codes, and local safe operating procedures or equivalent work is carried out in accordance with legislative obligations, environmental legislation, relevant health regulations, manual handling procedures and organisation insurance requirements work requires individuals to demonstrate some discretion, judgement and problem solving |
| Cut mat boards includes: | cutting of mat boards by hand or machine according to required shape in straight line or oval with bevel cuts for the mounting of artwork |
| Machines include: | straight line mat cutters circle/oval mat cutters manual, wall-mounted, pneumatic or computerised, and use interchangeable cutting heads |
| Tools and equipment include: | hand-operated tools: cutting blades clamps marking out tools application equipment manual and computerised mat cutters |
| Materials include: | mat boards and various forms of artwork, which may made be of: paper pulp rag/cotton pulp or alpha cellulose pulp board finishes: suede plain cotton or metallic |
| Personal protective equipment includes: | that prescribed under legislation, regulations and enterprise policies |
| Information and procedures include: | workplace procedures relating to the use of tools and equipment work instructions, including job sheets, cutting lists, plans, drawings and designs workplace procedures relating to reporting and communication manufacturer specifications and operational procedures |

# Unit Mapping Information

Supersedes and is equivalent to LMFPF2005B Cut mat boards.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.education.gov.au/Pages/TrainingDocs.aspx?q=0601ab95-583a-4e93-b2d4-cfb27b03ed73