# Assessment Requirements for MSFPF4004 Determine and apply gilding techniques

Release: 1

Assessment Requirements for MSFPF4004 Determine and apply gilding techniques

# Modification History

Release 1. Supersedes and is equivalent to MSFPF4002 Determine and apply gilding techniques.

# Performance Evidence

There must be evidence the candidate has completed the tasks outlined in the elements and performance criteria of this unit, and:

refurbished an existing gilded frame

used differing techniques and materials to prepare and gild 2 new frames.

# Knowledge Evidence

There must be evidence the candidate has knowledge of:

work flow process for gilding operations

safety considerations and requirements for gilding, including personal protective equipment

mathematical procedures for estimation, measurement and use of ratios for mixing

types of specific work instructions for gilding:

choice of substrate

surface preparation

selection of bole or poliment

selection and application of size

selection and application of leaf, patination and sealing of leaf

techniques and application of gilding, including:

oil

size

quick size (acrylic)

water gilding based on the reactivation of the glue in the bole layer

new and emerging techniques, such as Kolnar gilding

types of leaf used in gilding and their features:

gold

silver

platinum in varying alloys

schlagmetal (brass) in varying alloys

copper

aluminium

the role and nature of composition work

methods, characteristics and problems of application

functions, features and safe use of gilding tools and equipment:

gilder’s cushion

gilder’s knife

gilder’s tip

brushes

agate burnishing stones

materials used in gilding, their features and uses:

alloy and aluminium leaf

bole (wet or dry)

ethanol and brandy

gilding oil

glue

gold leaf

rosin

shellac

schlag metal (Dutch metal)

size (parchment or rabbit skin)

turpentine

varnish colours and chemicals for patination, such as potassium sulphide

whiting (chalk)

potential impacts of the gilding process on artwork and how to avoid damage

documentation requirements for identifying and recording work.

# Assessment Conditions

Skills must have been demonstrated in the workplace or in a simulated environment that reflects workplace conditions and contingencies. The following conditions must be met for this unit:

use of suitable facilities, equipment and resources, including:

workspace equipped with:

gilding tools, equipment and consumables to complete tasks in the Performance Evidence

personal protective equipment

work orders

items to be gild finished.

Assessors must satisfy the NVR/AQTF mandatory competency requirements for assessors.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0601ab95-583a-4e93-b2d4-cfb27b03ed73