# Assessment Requirements for MSFPT4013 Voice a piano

Release: 1

Assessment Requirements for MSFPT4013 Voice a piano

# Modification History

Release 1. Changes to code, application, elements, performance criteria, assessment requirements. Foundation skills added. Range of conditions removed. Assessment requirements changed.

# Performance Evidence

There must be evidence the candidate has completed all the tasks outlined in the elements and performance criteria of this unit, and demonstrated the ability to:

voice at least one upright or grand piano.

# Knowledge Evidence

There must be evidence the candidate has knowledge of:

workplace standards, requirements, policies and procedures for tuning and voicing pianos

knowledge of piano tuning and regulation methodologies

the nature of undesirable piano tonal qualities, including:

harsh/soft noise which may occur when there is a loose hammer head, loose centre pins or uneven level strings

string noises

false beats in strings

bright or metallic sound due to hard hammers

needling techniques, including:

correct method of supporting the hammers to avoid damage to action centres

selecting the appropriate needling zones to achieve particular tonal effects

hardening techniques, including:

lacquers

irons

collodion

types, characteristics, uses, limitations and preparation of materials and equipment used in piano tuning and voicing

types of tools and equipment used to tune and voice pianos and procedures for their safe use, operation and maintenance, including:

screwdrivers

voicing needles

voicing tools and blocks

hammer iron

lacquer and brush

sandpaper

hammer lifting tool

tuning wedge

string level tool

string lifting tool

problem identification for piano voicing and resolution

set-up and operation of tools and equipment used for tuning pianos

procedures for recording, reporting and maintaining workplace records and information

best physical and ergonomic approaches to promote muscle control and avoid fatigue, including:

standing or seated according to type of piano and height of individual

evenly distributing body weight between both feet while standing to reduce fatigue

use a resting arm, where available, to aid control during tuning and reduce tiredness

work health and safety requirements for piano tuning.

# Assessment Conditions

Skills must have been demonstrated in the workplace or in a simulated environment that reflects workplace conditions and contingencies. The following conditions must be met for this unit:

use of suitable facilities, equipment and resources required for performance evidence, including:

pianos for tuning

tools and materials used in piano tuning

workplace procedures, manufacturing specifications, codes, standards, manuals, and reference materials.

Assessors must satisfy the NVR/AQTF mandatory competency requirements for assessors.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0601ab95-583a-4e93-b2d4-cfb27b03ed73