# MSFSF1001 Produce basic soft furnishing accessories

Release: 1

MSFSF1001 Produce basic soft furnishing accessories

# Modification History

Release 1 - New unit of competency

# Application

This unit of competency covers producing basic soft furnishing accessories, such as basic cushions and other decorative items, following the requirements of patterns.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

# Competency Field

# Unit Sector

Soft Furnishing

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe the performance needed to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Identify work requirements | 1.1 | Job requirements are identified from work instructions and patterns |
|  |  | 1.2 | Work health and safety (WHS) requirements, including ergonomic criteria and personal protection needs, are observed throughout the work |
|  |  | 1.3 | Pattern is interpreted to determine tool, equipment and material requirements |
|  |  | 1.4 | Instructions for sequencing of work and particular measurements or adjustments are interpreted |
|  |  | 1.5 | Work sequence is planned, including quality checkpoints |
| 2 | Prepare for work | 2.1 | Suitable work area is selected for the tasks |
|  |  | 2.2 | Materials are selected to match customer requirements, suitability for the purpose and work instructions |
|  |  | 2.3 | Potential hazards are identified from observation of the tools, equipment, materials and workplace reference material |
|  |  | 2.4 | Required tools, equipment and materials are collected in the work area |
| 3 | Complete production | 3.1 | Soft furnishing accessories are produced following work instructions |
|  |  | 3.2 | Work is checked against patterns and worksheets at the identified checkpoints |
|  |  | 3.3 | Non-conformity with the required quality standards is rectified |
| 4 | Finalise operations | 4.1 | Completed work is checked against required quality standards |
|  |  | 4.2 | Documentation is completed following workplace procedures |
|  |  | 4.3 | Work area is cleaned, and tools and equipment are returned to storage |
|  |  | 4.4 | Unused materials are returned to storage and waste and scrap are dealt with following workplace procedures |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency. Detail on appropriate performance levels for each furnishing unit of competency in reading, writing, oral communication and numeracy utilising the Australian Core Skills Framework (ACSF) are provided in the Furnishing Training Package Implementation Guide.

# Range of Conditions

| Specifies different work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) are included. Range is restricted to essential operating conditions and any other variables essential to the work environment. |  |
| --- | --- |
| Unit context includes: | WHS requirements, including legislation, building codes, material safety management systems, hazardous and dangerous goods codes, and local safe operating procedures or equivalent work is carried out in accordance with legislative obligations, environmental legislation, relevant health regulations, manual handling procedures and organisation insurance requirements work requires individuals to demonstrate some discretion, judgement and problem solving |
| Soft furnishing accessories include: | basic cushions bolsters soft tableware and slip-on covers |
| Tools and equipment include: | measuring and calculating equipment: tapes rulers calculators computers fabric and general cutting and sewing tools: scissors saws drills staplers sewing machines hemming machines overlockers glue guns |
| Materials include: | plain and patterned fabric velvet vinyl piping cords wadding foam zips and fasteners |
| Pattern interpretation includes: | the determination of type of material specified, required tools and equipment for the task, number and type of soft furnishing accessories to be produced, and quality requirements |
| Personal protective equipment includes: | that prescribed under legislation, regulations and enterprise policies and practices |
| Information and procedures include: | workplace procedures relating to the use of tools and equipment work instructions, including job sheets, cutting lists, plans, drawings and designs workplace procedures relating to reporting and communication manufacturer specifications and operational procedures |

# Unit Mapping Information

Supersedes and is equivalent to LMFSF1001B Produce basic soft furnishing accessories.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0601ab95-583a-4e93-b2d4-cfb27b03ed73

Assessment Requirements for MSFSF1001 Produce basic soft furnishing accessories

# Modification History

Release 1 - New unit of competency

# Performance Evidence

Interpret work order and locate and apply relevant information

Apply safe handling requirements for equipment, products and materials, including use of personal protective equipment

Identify materials used in the work process

Follow work instructions, operating procedures and inspection processes to:

minimise the risk of injury to self or others

prevent damage to goods, equipment and products

maintain required production output and product quality

Interpret patterns and produce at least one (1) type of soft furniture accessory

Use mathematical ideas and techniques to correctly complete measurements, calculate area and estimate material requirements

Communicate ideas and information to enable confirmation of work requirements and specifications and the reporting of work outcomes and problems, interpret basic plans and follow safety procedures

Avoid backtracking, work flow interruptions or wastage

Work with others and in a team by recognising dependencies and using cooperative approaches to optimise work flow and productivity

# Knowledge Evidence

Types, characteristics, uses and limitations of basic soft furnishing materials

Interpretation of patterns

Identification of equipment, processes and procedures

Work flow in relation to soft furnishing accessory production

# Assessment Conditions

Assessors must:

hold training and assessment competencies as determined by the National Skills Standards Council (NSSC) or its successors

have vocational competency in the furnishing industry at least to the level being assessed with broad industry knowledge and experience, usually combined with a relevant industry qualification

be familiar with the current skills and knowledge used and have relevant, current experience in the furnishing industry.

Assessment methods must confirm consistency of performance over time rather than a single assessment event and in a range of workplace relevant contexts.

Assessment must be by observation of relevant tasks with questioning on underpinning knowledge and, where applicable, multimedia evidence, supervisor’s reports, projects and work samples.

Assessment is to be conducted on single units of competency or in conjunction with other related units of competency. Foundation skills are integral to competent performance in the unit and should not be assessed separately.

Assessment must occur on the job or in a workplace simulated facility with relevant process, equipment, materials, work instructions and deadlines.

Access is required to soft furnishing accessory patterns and materials.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0601ab95-583a-4e93-b2d4-cfb27b03ed73