# MSFSF2012 Fit and adjust interior blinds

Release: 1

MSFSF2012 Fit and adjust interior blinds

# Modification History

Release 1 - New unit of competency

# Application

This unit of competency covers fitting and adjusting interior blinds.

Licensing, legislative or certification requirements may apply to this unit and relevant state/territory and local government agencies should be consulted to determine any necessary certification or licensing for undertaking interior blinds work. Access to construction sites requires certification of general induction training specified by the National Code of Practice for Induction for Construction Work (ASCC 2007).

# Pre-requisite Unit

# Competency Field

# Unit Sector

Soft Furnishing

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe the performance needed to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Identify products, fittings and attachments to be used | 1.1 | Type of blind to be fitted and work order requirements are identified |
|  |  | 1.2 | Attachments, required materials and tools are identified from work instructions and, where necessary, site inspection |
|  |  | 1.3 | The effect of the fitting process on overall finished window is identified |
| 2 | Plan process for fitting blinds | 2.1 | Work health and safety (WHS) requirements, including ergonomic criteria and personal protection needs, are observed throughout the work |
|  |  | 2.2 | Appropriate equipment is identified to minimise risks of damage to customers’ premises or injuries to self |
|  |  | 2.3 | Manufacturer information on products used is located and used to plan work |
|  |  | 2.4 | Attachment process is planned to conform to quality requirements, maximise the appearance of the finished work and provide for the most economical use of materials and labour |
|  |  | 2.5 | Tools, equipment and materials are selected and checked prior to use to ensure that they are appropriate for the work, serviceable and in a safe condition |
|  |  | 2.6 | Blind operating mechanism is set up and checked against workplace requirements and work instructions |
| 3 | Complete fitting operations | 3.1 | Work plan is followed ensuring compliance with workplace procedures and WHS requirements |
|  |  | 3.2 | Blinds are fitted and tested for correct operation |
|  |  | 3.3 | Pelmets, cord brackets or pulleys are fitted using appropriate hardware and adjusted to suit blind operation |
| 4 | Clean work area | 4.1 | Worksite and any equipment used are cleaned and stored appropriately |
|  |  | 4.2 | Unserviceable equipment is tagged, faults are identified and appropriate personnel are informed |
|  |  | 4.3 | Completed work is inspected and compared to workplace quality requirements |
|  |  | 4.4 | Blinds which do not meet quality specifications are repaired on-site or returned to the workroom for repair |
|  |  | 4.5 | Appropriate documentation is completed |
|  |  | 4.6 | Fittings and blinds which do not meet quality specifications are repaired on-site or returned to the workroom for repair |
|  |  | 4.7 | Appropriate documentation/reporting is completed |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency. Detail on appropriate performance levels for each furnishing unit of competency in reading, writing, oral communication and numeracy utilising the Australian Core Skills Framework (ACSF) are provided in the Furnishing Training Package Implementation Guide.

# Range of Conditions

| Specifies different work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) are included. Range is restricted to essential operating conditions and any other variables essential to the work environment. |  |
| --- | --- |
| Unit context includes: | WHS requirements, including legislation, building codes, material safety management systems, hazardous and dangerous goods codes, and local safe operating procedures or equivalent work is carried out in accordance with legislative obligations, environmental legislation, relevant health regulations, manual handling procedures and organisation insurance requirements work requires individuals to demonstrate some discretion, judgement and problem solving |
| Blinds to be fitted include: | Roman Austrian festoon Holland Venetian vertical pleated |
| Tools and equipment include: | hand and power tools hammers screwdrivers drills |
| Personal protective equipment includes: | that prescribed under legislation, regulations and enterprise policies and practices |
| Information and procedures include: | workplace procedures relating to the use of tools and equipment work instructions, including job sheets, cutting lists, plans, drawings and designs workplace procedures relating to reporting and communication manufacturer specifications and operational procedures |

# Unit Mapping Information

Supersedes and is equivalent to LMFSF2012B Fit and adjust interior blinds.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0601ab95-583a-4e93-b2d4-cfb27b03ed73