# MSFSF2015 Cut single layer fabrics

Release: 1

MSFSF2015 Cut single layer fabrics

# Modification History

Release 1. Supersedes and is equivalent to MSFSF2001 Cut single layer fabrics

# Application

This unit of competency describes the skills and knowledge required to measure, mark out and cut single layer fabrics for the manufacture of curtains and soft furnishing accessories.

The unit applies to those in the furnishing industry cutting all types of single layer fabrics, including lining and interlining. It does not apply to cutting from patterns.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Not applicable.

# Unit Sector

Soft furnishing

# Elements and Performance Criteria

| Elements Elements describe the essential outcomes. | Performance Criteria Performance criteria describe the performance needed to demonstrate achievement of the element. |  |
| --- | --- | --- |
| 1. Prepare to cut single layer fabrics | 1.1 | Identify job and equipment requirements from work order |
|  | 1.2 | Select and lay out fabrics according work order and workplace procedures |
|  | 1.3 | Inspect fabrics for flaws, design features affecting job requirements and required finish |
|  | 1.4 | Select suitable scissors and blades and check prior to use for required sharpness, set, operation and safe condition |
| 2. Undertake cutting process | 2.1 | Mark out measurements for pleat lines, hems, headings and special seam allowances using workplace-approved methods |
|  | 2.2 | Cut fabric according to work health and safety (WHS) requirements, job requirements, workplace procedures and material characteristics |
|  | 2.3 | Hang fabric using required protective covers and attached care label information |
| 3. Finish and dispatch work | 3.1 | Check that completed work complies with quality and work order requirements, and rectify identified deficiencies within scope of own role |
|  | 3.2 | Dispatch cut lengths according to workplace procedures |
|  | 3.3 | Collect and store material that can be re-used, and dispose of waste safely and sustainably according to workplace procedures |
|  | 3.4 | Clean work area and equipment, and tag damaged tools and equipment |
|  | 3.5 | Complete required workplace records according to workplace procedures |

# Foundation Skills

| This section describes those language, literacy, numeracy and employment skills that are essential to performance but not explicit in the performance criteria of this unit of competency. |  |
| --- | --- |
| Numeracy skills to: | interpret, discuss and use mathematical information in routine workplace documentation use mathematical information to complete measurements, calculate area and estimate material requirements |
| Reading skills to: | interpret routine workplace documentation, including work orders, WHS requirements and workplace installation procedures |

# Unit Mapping Information

Release 1. Supersedes and is equivalent to MSFSF2001 Cut single layer fabrics

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0601ab95-583a-4e93-b2d4-cfb27b03ed73