# MSFSF2022 Cut single layer fabrics

Release: 1

MSFSF2022 Cut single layer fabrics

# Modification History

Release 1. Changes to code, application, elements, performance criteria, assessment requirements. Foundation skills added. Range of conditions removed. Assessment requirements changed.

# Application

This unit describes the skills and knowledge required to measure, mark out and cut single layer fabrics for the manufacture of curtains and soft furnishing accessories. It does not apply to cutting from patterns.

Soft furnishing workers with this competency follow established procedures.

No licensing or certification requirements exist at the time of publication. Relevant legislation, industry standards and codes of practice within Australia must be applied.

# Competency Field

Soft furnishing

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Establish task requirements and prepare for work | 1.1 Determine task requirements from work documentation 1.2 Identify and follow work health and safety requirements 1.3 Select and lay out fabrics to meet task requirements 1.4 Inspect fabrics for flaws and design features affecting task requirements and required finish and make adjustments as needed 1.5 Select scissors and blades and check they are safe, functional and sharp |
| 2. Undertake cutting process | 2.1 Mark out measurements for pleat lines, hems, headings and special seam allowances using workplace procedures 2.2 Ensure mark out minimises wastage while meeting job requirements 2.3 Cut fabric to meet task requirements 2.4 Hang fabric using required protective covers and according to care label information |
| 3. Finish and dispatch work | 3.1 Check that completed work complies with task quality requirements 3.2 Rectify identified deficiencies within scope of own role 3.3 Dispatch cut lengths according to workplace procedures 3.4 Collect and store material that can be re-used, and dispose of waste according to workplace procedures 3.5 Clean work area and equipment, and tag damaged tools and equipment 3.6 Complete required workplace records according to workplace procedures |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential to performance but not explicit in the performance criteria.

Numeracy skills to measure and calculate lengths and areas and estimate material requirements

Reading skills to interpret familiar workplace procedures and work documentation

# Unit Mapping Information

Release 1. Supersedes and is equivalent to MSFSF2015 Cut single layer fabrics

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=ce216c9c-04d5-4b3b-9bcf-4e81d0950371