# Assessment Requirements for MSFSF3015 Construct Roman-style blinds

Release: 1

Assessment Requirements for MSFSF3015 Construct Roman-style blinds

# Modification History

Release 1. Changes to code, application, elements, performance criteria, assessment requirements. Foundation skills added. Range of conditions removed. Assessment requirements changed.

# Performance Evidence

There must be evidence the candidate has completed the tasks outlined in the elements and performance criteria of this unit, and demonstrated the ability to:

safely measure and construct at least two different types of Roman-style blinds to meet task and workplace size and quality requirements. These blinds must differ in size and style and have different types of headers, pull-up battens, and fabric type and design.

This work must meet workplace and industry standards for efficiency, including:

identifying and resolving routine problems and faults, if encountered

working with others using cooperative approaches to optimise workflow and productivity.

# Knowledge Evidence

There must be evidence the candidate has knowledge of:

how to identify and interpret key information from work documentation for the construction of Roman-style blinds, including work orders, cutting lists, designs, and supplier instructions and specifications

processes and quality requirements for Roman-style blind construction, including:

headboard and track

cutting patterns and sequences

measuring and marking out fold or stitching lines

number and widths of batten spacings

laying out lining and interlining materials and folding ready for sewing

matching and securing lining material patterns

checking pattern matching and maintaining finished length

keeping pattern matches uniform within drops and across matching sets of blinds

machine sewing operations, including even and appropriate seam stitch type, length, and visibility

inserting and fixing batten pockets

required weights, stitch type and sewing method for hems

end finishing

fabric adhesion and fasteners

inspection of finished blind

disposing of waste safely and in an environmentally sustainable manner

using, cleaning, and storing tools and equipment

tagging and reporting faulty tools and equipment

recycling and re-use

recording and reporting blind construction information

function of tools and components used to construct Roman-style blinds:

materials:

headboard and track

fabrics

threads

power tools:

sewing machines

drills

pressing equipment

saws

staplers

hardware:

needle types suited to fabric used in the performance evidence

aluminium and timber battens

screws, brackets and assembly hardware

how to identify and mitigate work process problems within scope of own role

risks commonly encountered in constructing Roman-style blinds, including risks to health from repetitive actions and manual handling

risk mitigation methods, including personal protective equipment (PPE)

workplace communication channels and protocols

common fabric faults and problems, and how to respond

# Assessment Conditions

Skills must have been demonstrated in the workplace or in a simulated environment that reflects workplace conditions and contingencies. The following conditions must be met for this unit:

use of suitable facilities, equipment and resources, including:

fabrics to construct Roman-style blinds

tools and equipment to construct Roman-style blinds, including sewing machine and PPE

workplace documentation required for blind-construction process, including:

work orders

workplace procedures

equipment and supplier instructions

manufacturer specifications

work health and safety documentation

# Links

Companion Volume implementation guides are found in VETNet – - https://vetnet.education.gov.au/Pages/TrainingDocs.aspx?q=0601ab95-583a-4e93-b2d4-cfb27b03ed73