# Assessment Requirements for MSFUP3017 Fit and apply final cover materials to furniture using decorative techniques

Release: 1

Assessment Requirements for MSFUP3017 Fit and apply final cover materials to furniture using decorative techniques

# Modification History

Release 1. Changes to code, application, elements, performance criteria, assessment requirements. Foundation skills added. Range of conditions removed. Assessment requirements changed.

# Performance Evidence

There must be evidence the candidate has completed all the tasks outlined in the elements and performance criteria of this unit, and demonstrated the ability to:

fit and apply a final material cover for:

2 bedheads of at least 700 mm x 500 mm, using diamond buttoning, one with hand pleating, one with stitched pleating

a chair, using other decorative finishes, including one or more of the following:

pleating

fluting

stab and deep buttoning (elongated).

# Knowledge Evidence

There must be evidence the candidate has knowledge of:

work requirements, including workplace quality standards, for fitting and applying final covers with decorative finishes

workflows for applying final material covers to upholstered furniture using decorative techniques

characteristics, common faults and inspection procedures for material used for final covers, including:

plain and patterned fabrics

vinyl

velvet

the function and characteristics of items used in applying final covers to furniture, including at least 5 of the following:

piping

cords

pleats

glues

staples

buttons

tacks

tingles/gimp pins

antique nails/upholstery studs

quality requirements for applying final cover materials, including ensuring that:

patterns, naps/pile, sheens, direction of pattern, pattern repeat, and finished side of the materials are positioned appropriately

the finish of the material over the furniture foundation is smooth and consistent

weaving lines are straight and square to each other

tools commonly used when applying final cover materials to furniture

work health and safety requirements, including personal protective equipment (PPE) and manual handling procedures.

# Assessment Conditions

Skills must have been demonstrated in the workplace or in a simulated environment that reflects workplace conditions and contingencies. The following conditions must be met for this unit:

use of suitable facilities, equipment and resources, including:

work orders

cover materials, trims, furniture frames and foundations, fixing materials and tools required for performance evidence, including PPE

workplace procedures, manufacturing specifications, codes, standards, manuals, and reference materials.

Assessors must satisfy the NVR/AQTF mandatory competency requirements for assessors.

# Links

Companion Volume implementation guides are found in VETNet – - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0601ab95-583a-4e93-b2d4-cfb27b03ed73