# Assessment Requirements for MSFUP3021 Manufacture and fit loose covers

Release: 1

Assessment Requirements for MSFUP3021 Manufacture and fit loose covers

# Modification History

Release 1. Changes to code, application, elements, performance criteria, assessment requirements. Foundation skills added. Range of conditions removed. Assessment requirements changed.

# Performance Evidence

There must be evidence the candidate has completed the tasks outlined in the elements and performance criteria of this unit, and demonstrated the ability to:

develop 2 patterns for loose covers for given items of furniture, including:

one plain material, including piping detail

one patterned material with inverted pleat skirt, including piping detail.

# Knowledge Evidence

There must be evidence the candidate has knowledge of:

work requirements, including workplace standards and workflows for manufacturing and fitting loose covers

fabric types, including plain, patterned and velvet, and common faults and inspection procedures

design features of the finished furnishing items and how this relates to laying out the pattern and cutting the fabrics

sewing machines typically required to make loose covers, including:

single or multi-needle flatbed

walking foot

overlocker and gatherer

how to set up and adjust sewing machine features, including:

threading machine and winding bobbin

setting required tensions

selecting required machine settings

lubricating and adjusting the machine

testing operations for work order requirements

work health and safety requirements, including personal protective equipment (PPE) and manual handling procedures.

# Assessment Conditions

Skills must have been demonstrated in the workplace or in a simulated environment that reflects workplace conditions and contingencies. The following conditions must be met for this unit:

use of suitable facilities, equipment and resources, including:

work orders

plain and patterned fabrics, sewing machines and furniture to be upholstered for performance evidence

PPE

workplace procedures, manufacturing specifications, codes, standards, manuals, and reference materials.

Assessors must satisfy the NVR/AQTF mandatory competency requirements for assessors.

# Links

Companion Volume implementation guides are found in VETNet – - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0601ab95-583a-4e93-b2d4-cfb27b03ed73