# Assessment Requirements for MSL925003 Determine measurements of uncertainty

Release: 1

Assessment Requirements for MSL925003 Determine measurements of uncertainty

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version was released in MSL Laboratory Operations Training Package Release 2.0. Supersedes and equivalent to MSL925002 Analyse measurements and estimate uncertainties. Prerequisite removed. Range of conditions removed. Assessment requirements amended. |

# Performance Evidence

There must be evidence the candidate has completed the tasks outlined in the elements and performance criteria of this unit, and:

analysed measurements and estimated uncertainties of a minimum of 3 sets of data, including:

using scientific notation, correct units and correct number of significant figures

evaluating formulae containing powers, exponents and logarithms functions

preparing and interpreting linear graphs

using statistical analysis to estimate and report measurement uncertainty in accordance with the ISO Guide to the Expression of Uncertainty in Measurement

calculating a combined standard uncertainty using root-sum-of-squares, accounting for correlations where necessary

gathering information about uncertainty components from calibration reports or reference material report

calculating sensitivity coefficients either experimentally or by partial differentiation

reporting results and uncertainties in the required formats.

# Knowledge Evidence

There must be evidence the candidate has knowledge of:

the steps in the measurement, test or calibration involved

the difference between errors, corrections and uncertainties

uncertainty in the uncertainty estimation process

uncertainty components:

calibration uncertainty

instability or drift in the calibrated instrument

repeatability of the results

resolution or readability of the instrument

environmental influences, such as temperature, air pressure, humidity, vibration, electrical noise and gravity

reference material uncertainty

common to the use of an instrument

factors arising from the instrument being used under different conditions to those when it was calibrated

factors arising from using an instrument under a different operating environment or procedures (such as orientation of a transducer and immersion depth of a temperature probe)

reproducibility of quality control data

workplace procedures for:

determining the uncertainty components associated with each of the inputs and whether they are significant and for applying appropriate corrections

determining uncertainty components from quality control data

manufacturer’s specifications (including instrument drift specification and reference materials)

the concept of degrees of freedom

the characteristics of a valid measurement

reporting requirements: the uncertainty in measurement (GUM), National Association of Testing Authorities (NATA) or other applicable reference material

Australian and international standards, codes and guides as they relate to the job role

legal, ethical and work health and safety (WHS) requirements specific to the work task.

# Assessment Conditions

Skills must have been demonstrated in the workplace or in a simulated environment that reflects workplace conditions and contingencies. The following conditions must be met for this unit:

use of suitable facilities, equipment and resources, including:

data sets and records

test methods and description of test set-up

computer and relevant software or laboratory information system

workplace sampling and test procedures.

Assessors must satisfy the NVR/AQTF mandatory competency requirements for assessors.

# Links

MSL Laboratory Operations Companion Volume Implementation Guide is available from VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=5c63a03b-4a6b-4ae5-9560-1e3c5f462baa