# Assessment Requirements for MSL954002 Prepare mineral samples for analysis

Release: 1

Assessment Requirements for MSL954002 Prepare mineral samples for analysis

# Modification History

Release 1. Supersedes and is equivalent to MSL954002A Prepare mineral samples for analysis

# Performance Evidence

Evidence of competence in this unit must satisfy all of the requirements of the elements and performance criteria, and include demonstration of:

safely reducing at least three (3) types of mineral samples to representative samples and preparing analytical portions of each that meet client requirements for analysis:

representative

free of contamination

of specified quantity and particle size

ready for analysis

interpreting and closely following preparation methods

achieving the required sample throughput

recognising problems and invalid preparation steps and taking appropriate corrective actions

recognising limitations and seeking timely advice

minimising rework, waste and environmental impact

completing preparation and production documentation accurately

maintaining equipment/reagents and disposing of all waste, surplus and spent samples according to workplace procedures

recognising hazards and using specified safety equipment and safe work procedures at all times.

# Knowledge Evidence

Must provide evidence that demonstrates knowledge of:

geological properties of samples used in job role, including sulphides, oxides and silicates

terminology, including homogeneous, heterogeneous, integrity and segregation

distribution of common analytes in a matrix

chemical reactions associated with common preparation methods used in job role

effects of reagents on the element of interest

reaction and recovery rates, solubility and equilibria

tracking analytes of interest during changes of state

critical preparation steps that determine analytical accuracy and precision, including:

monitoring drying (incipient and total)

mixing to ensure homogeneity before sub-sampling

suitability of reagents for purpose (e.g. dryness)

accurate operation of dispensers and balances

critical/non-critical volumes and critical reagent quantities

temperature control during digests

loss of solution prior to/after mixing

type and acid strength in final solutions

mechanical loss of digest (sputtering, residues on glassware/plastic ware and filtering)

function of key components and operating procedures for sample preparation equipment

calculation steps in preparation methods (e.g. serial dilution)

use of non-international system of units (SI) units (ppm, ppb) and SI units, and conversions

workplace and/or legal traceability requirements

relevant hazards, work health and safety (WHS) and environmental requirements, including use of material safety data sheets (MSDS) and antidotes for specific hazards, such as hydrofluoric acid and cyanide.

# Assessment Conditions

Judgement of competence must be based on holistic assessment of the evidence. Assessment methods must confirm consistency of performance over time, rather than a single assessment event.

This unit of competency is to be assessed in the workplace or a simulated workplace environment. A simulated workplace environment must reflect realistic operational workplace conditions that cover all aspects of workplace performance, including the environment, task skills, task management skills, contingency management skills and job role environment skills.

Foundation skills are integral to competent performance of the unit and should not be assessed separately.

Assessment processes and techniques must be appropriate to the language, literacy and numeracy requirements of the work being performed and the needs of the candidate.

Knowledge evidence may be collected concurrently with performance evidence or through an independent process, such as workbooks, written assessments or interviews (provided a record is kept in each case).

This unit of competency may be assessed with:

MSL924001 Record and present data

MSL943002 Participate in laboratory or field workplace safety

Holistic assessment methods include:

sizing checks and grind performance for samples prepared by candidate

review of quality control performance and analytical results traceable to samples prepared by the candidate

review of preparation and production documentation prepared by the candidate

feedback from supervisors and/or clients about the candidate’s ability to safely prepare mineral samples that meet client requirements

observation of the candidate preparing mineral samples for analysis

oral or written questions about preparation methods, critical steps, typical problems and corrective actions.

Access is required to instruments, equipment, materials, workplace documentation, procedures and specifications associated with this unit, including, but not limited to:

a variety of mineral samples, sample preparation methods, reagents, sample containers and labels

client requests/documentation, such as client profile; sample identification and sample receipts; required preparation methods, storage and analyses; and service charges

sample preparation equipment, such as splitters (e.g. riffles and rotary dividers), mills (e.g. ball, ring and rod), bowls (e.g. chrome-steel, tungsten-carbide and zirconia) and tumblers, crushers (e.g. cone, jaw and roll), grinders and disc pulverisers, sieves, ovens and muffle furnaces

laboratory equipment, such as ultrasonic baths, centrifuges and vacuum and pressure filtration, volumetric glassware/plastic ware and dispensers, analytical balances and auto-samplers

safety equipment and safe work procedures.

Assessors must satisfy the assessor competency requirements that are in place at the time of the assessment as set by the VET regulator.

The assessor must demonstrate both technical competence and currency.

Technical competence can be demonstrated through:

relevant VET or other qualification/Statement of Attainment AND/OR

relevant workplace experience.

Currency can be demonstrated through:

performing the competency being assessed as part of current employment OR

having consulted with a laboratory about performing the competency being assessed within the last twelve months.

# Links

MSA Training Package Implementation Guides - http://mskills.org.au/training-packages/info/