# MSL973007 Perform microscopic examination

Release: 1

MSL973007 Perform microscopic examination

# Modification History

Release 1. Supersedes and is equivalent to MSL973007A Perform microscopic examination

# Application

This unit of competency covers the ability to set up a light microscope for optimum resolution, to prepare routine samples and to observe, identify and report sample characteristics.

This unit of competency is applicable to laboratory or technical assistants in all industry sectors. The unit of competency covers limited interpretation and analysis of results. Troubleshooting of equipment and procedures is not required.

While no specific licensing or certification requirements apply to this unit at the time of publication, laboratory operations are governed by relevant legislation, regulations and/or external accreditation requirements. Local requirements should be checked.

# Pre-requisite Unit

Nil

# Competency Field

Testing

# Unit Sector

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe the performance needed to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Interpret test requirements | 1.1 | Review test request to identify samples to be tested, test method and equipment involved |
|  |  | 1.2 | Identify hazards associated with the sample, preparation methods and equipment, and implement workplace control measures |
| 2 | Set up work area for preparation and examination of samples | 2.1 | Collect equipment and arrange the workspace so that equipment can be used safely and efficiently |
|  |  | 2.2 | Perform pre-use and safety checks to ensure equipment is fit for purpose and report faulty or unsafe equipment to appropriate personnel |
| 3 | Prepare samples for examination | 3.1 | Log and label samples according to workplace procedures to ensure traceability |
|  |  | 3.2 | Check suitability of the original and prepared sample for the examination, and report unsuitable samples to appropriate personnel |
|  |  | 3.3 | Prepare and store the sample for examination following workplace methods |
| 4 | Set up and use a light microscope | 4.1 | Set up the light path to optimise resolution |
|  |  | 4.2 | Select the appropriate objectives and filter for the sample being examined |
|  |  | 4.3 | Ensure that the lenses are clean |
|  |  | 4.4 | Adjust settings and alignment of the light path to optimise performance |
|  |  | 4.5 | Place sample correctly on the stage |
| 5 | Observe, identify and report sample characteristics | 5.1 | Recognise and identify significant sample characteristics |
|  |  | 5.2 | Perform required calculations accurately |
|  |  | 5.3 | Prepare and view control samples and check that results are consistent with expected values |
|  |  | 5.4 | Identify and report out-of-specification or atypical results promptly to appropriate personnel |
|  |  | 5.5 | Record and report data in accordance with workplace procedures |
| 6 | Maintain a safe work environment | 6.1 | Ensure safety and minimise cross-contamination through the use of personal protective equipment (PPE) |
|  |  | 6.2 | Handle all samples and equipment in accordance with workplace safety protocols |
|  |  | 6.3 | Clean up spills using appropriate techniques to protect personnel, work area and environment |
|  |  | 6.4 | Minimise generation of waste and environmental impacts |
|  |  | 6.5 | Collect and dispose of all wastes safely |
|  |  | 6.6 | Report hazards and incidents to designated personnel using workplace procedures |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential to performance.

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Range of Conditions

| This field allows for different work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) are included. |  |
| --- | --- |
| Standards, codes, procedures and/or workplace requirements | Standards, codes, procedures and/or workplace requirements include the latest version of one or more of: Australian and international standards covering the requirements for the competence of testing and calibration laboratories, laboratory safety and quality management national work health and safety (WHS) standards and codes of practice, and national measurement regulations and guidelines specific codes, guidelines and procedures, such as National Association of Testing Authorities (NATA) accreditation requirements, principles of good laboratory practice (GLP), and Australian code of good manufacturing practice for medicinal products (GMP) workplace documents, such as standard operating procedures (SOPs); quality and equipment manuals; calibration and maintenance schedules; material safety data sheets (MSDS) and safety procedures; material, production and product specifications; production and laboratory schedules; workplace recording and reporting procedures; and waste minimisation and safe disposal procedures workplace procedures for microscopic examination of samples |
| Preparation of samples | Preparation of samples includes, but is not limited to, one or more of: drying and cooling physical separation, centrifugation, filtration and chemical separation sub-sampling labelling aseptic transfer of specimen thin film or smear on a slide fixing of films to minimise cell damage and the production of artefacts staining of fixed material to illustrate required characteristics mounting of stained films, sections and whole mounts to ensure long-term preservation permanent labels for smears, films and sections for presentation, storage and retrieval selection of diluent to preserve or enhance visibility of the cells to be counted serial dilution to enable individual cells to be reliably counted filling a counting chamber in one continuous flow without bubbles or overflow selection, filling and cover slipping of a clean, dry counting chamber to ensure even distribution of cells during filling |
| Biological samples | Biological samples include, but are not limited to, one or more of: smears, impression smears, sections, squashes, films and whole mounts a monolayer of cells in smears and films fixed smears for demonstration of bacteria by the methylene blue and Gram staining techniques blood films stained by a Romanowsky technique to clearly show differentiation of granulocytes stained sections of animal tissues using regressive haematoxylin and eosin to differentiate cytoplasmic and nuclear detail differentially stained monocotyledon and dicotyledon stem sections to demonstrate the structure of vascular bundles (xylem, phloem and cambium) stained whole mounts of helminths whole mounts, such as liver flukes, planaria and samples of animal faeces to demonstrate ova, cysts and larvae pond water organisms onion root tip squash midstream sample of urine |
| Physical samples | Physical samples include, but are not limited to, one or more of: sand asbestos fibres coal samples construction materials for testing geological specimens |
| Sample characteristics | Sample characteristics are restricted to what can be viewed by bright light microscopy and include, but are not limited to, one or more of: shape and size of particles presence of contamination colour consistency and variability number of cells (e.g. cells in blood or other particulate samples, such as a yeast suspension or pollen grains) type of cells, percentage of atypical cells, presence/absence of cells, size of cells, viable and non-viable cells and trajectory presence of stained material, such as starch colour/staining and morphology motility |
| Workplace safety procedures | Workplace safety procedures include, but are not limited to, one or more of: ergonomic layout, correct illumination and organisation of workbench use of biohazard containers and laminar flow cabinet correctly labelling reagents and hazardous materials use of PPE, such as safety glasses, gloves and coveralls handling and storing hazardous materials and equipment in accordance with labels, MSDS and manufacturer instructions regularly cleaning and decontaminating equipment and work areas |
| WHS and environmental management requirements | WHS and environmental management requirements include:  complying with WHS and environmental management requirements at all times, which may be imposed through state/territory or federal legislation. These requirements must not be compromised at any time  applying standard precautions relating to the potentially hazardous nature of samples  accessing and applying current industry understanding of infection control issued by the National Health and Medical Research Council (NHMRC) and State and Territory Departments of Health, where relevant |

# Unit Mapping Information

Release 1. Supersedes and is equivalent to MSL973007A Perform microscopic examination

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=5c63a03b-4a6b-4ae5-9560-1e3c5f462baa