# Assessment Requirements for MSL974014 Classify soils

Release: 1

Assessment Requirements for MSL974014 Classify soils

# Modification History

Release 1. New unit

# Performance Evidence

Evidence of competence in this unit must satisfy all of the requirements of the elements and performance criteria, and include demonstration of:

correctly classifying at least two (2) different samples of each of the following:

gravel

sand

silt

clay

accurately interpreting job requests, client specifications and the relevant requirements of AS 1726-1993 Geotechnical site investigations, or its replacement

recognising soil characteristics and logging an accurate soil description that supports the classification given to the sample

interpreting test results and field observations with close attention to detail and making logical conclusions

reading graphs and tables to extract classification data, such as D10, D30 and D60 points

comparing the physical properties of samples with test results and expected values and recognising anomalous results

safely conducting laboratory tests to resolve anomalous results

performing accurate calculations, such as coefficients of curvature and uniformity for gravels and sands

seeking timely advice to deal with situations beyond own technical competence

providing classification results within the expected timeframe

maintaining accurate and complete records and confidentiality of client information

efficiently setting up, checking, using and cleaning/maintaining tools and equipment

collecting, storing and disposing of waste in accordance with workplace procedures.

# Knowledge Evidence

Must provide evidence that demonstrates knowledge of:

terms, symbols and requirements of AS 1726-1993 Geotechnical site investigations, or its replacement, used to perform soil classifications as part of job role

common purposes and client requirements for soil classifications conducted as part of job role

differences between classification of soils for agricultural and engineering purposes

common causes of misclassification of soils, such as:

identification of hard clay as sand or gravel

clayey gravels can have high plasticity

non-representative or contaminated samples

selecting inappropriate tests (e.g. clay-DCP and sand-penetrometer)

testing methods used for soil classifications as part of job role

relevant hazards, work health and safety (WHS) and environment requirements.

# Assessment Conditions

Judgment of competence must be based on holistic assessment of the evidence. Assessment methods must confirm consistency of performance over time, rather than a single assessment event.

This unit of competency is to be assessed in the workplace or a simulated workplace environment. A simulated workplace environment must reflect realistic operational workplace conditions that cover all aspects of workplace performance, including the environment, task skills, task management skills, contingency management skills and job role environment skills.

Foundation skills are integral to competent performance of the unit and should not be assessed separately.

Assessment processes and techniques must be appropriate to the language, literacy and numeracy requirements of the work being performed and the needs of the candidate.

Knowledge evidence may be collected concurrently with performance evidence or through an independent process, such as workbooks, written assessments or interviews (provided a record is kept in each case).

This unit of competency may be assessed with:

MSL974012 Perform tests to determine the properties of construction materials

Holistic assessment methods include:

review of soil classifications completed by the candidate

feedback from peers and supervisors about the candidate’s ability to conduct soil classifications safely, efficiently and in accordance with work instructions

examples of workplace documentation completed by the candidate such as records and calculations

observation of the candidate performing soil classification tasks

oral or written questioning about soil classification procedures, test methods, common soil classification problems and interpretation of site data.

Access is required to instruments, equipment, materials, workplace documentation, procedures and specifications associated with this unit, including, but not limited to:

access to job requests, client requirements, relevant site documentation and samples

workplace procedures, standards, test methods and equipment manuals

equipment and materials specified in test methods

safety clothing and equipment, including helmets, boots, gloves, earmuffs and glasses.

Assessors must satisfy the assessor competency requirements that are in place at the time of the assessment as set by the VET regulator.

The assessor must demonstrate both technical competence and currency.

Technical competence can be demonstrated through:

relevant VET or other qualification/Statement of Attainment AND/OR

relevant workplace experience.

Currency can be demonstrated through:

performing the competency being assessed as part of current employment OR

having consulted with a laboratory about performing the competency being assessed within the last twelve months.

# Links

MSA Training Package Implementation Guides - http://mskills.org.au/training-packages/info/