# Assessment Requirements for MSL974025 Prepare tissue and cell cultures

Release: 1

Assessment Requirements for MSL974025 Prepare tissue and cell cultures

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version was released in MSL Laboratory Operations Training Package Release 2.0. Supersedes and equivalent to MSL974011 Prepare tissue and cell cultures. Foundation skill information added. Range of conditions removed. Assessment requirements amended. |

# Performance Evidence

There must be evidence the candidate has completed the tasks outlined in the elements and performance criteria of this unit, and:

safely preparing at least 3 different primary tissue/cell cultures that are free of contamination using tissue/cell culture and basic subculture procedures, including:

manipulating equipment and materials and samples to prevent contamination at all preparation stages

performing routine maintenance and checks on raw materials and consumables, including use-by-date, possible contamination and storage conditions

growing cell lines and tissue to specifications without contaminating the original sample and the environment.

# Knowledge Evidence

There must be evidence the candidate has knowledge of:

basic classes and classification of culturable material including microorganisms, plant/animal cells and tissue

structure and function of plant/animal cells and organelles

cell structure, physiology and processes, including simple and facilitated diffusion, plasmolysis, osmosis, tonicity, active transport, energy production, mitosis, motility, phagocytosis and pinocystosis

concepts and principles of cell growth, including the need for nutrients, the role of growth regulators and the removal of wastes

types and sources of contamination

importance of strict aseptic techniques and cleaning procedures

common applications of plant tissue/cell culture

common applications of animal tissue/cell culture

typical methods for sterilisation and disposal of biohazardous:

steam and high-pressure air or steam

boiling, microwaving and autoclaving

filtration

gas, chemical and radiation

typical methods to preparing a primary culture

suitable culture conditions

subculture

quality control checks and quality assurance procedures

standard methods for preparing culture media, and operation and maintenance manuals for automated media preparation equipment

workplace and/or legal traceability requirements

awareness of environmental sustainability issues as they relate to the work task

legal, ethical and work health and safety (WHS) requirements specific to the work task.

# Assessment Conditions

Skills must have been demonstrated in the workplace or in a simulated environment that reflects workplace conditions and contingencies. The following conditions must be met for this unit:

use of suitable facilities, equipment and resources, including:

a laboratory equipped with appropriate equipment, samples, cell lines and reagents

tissue culture equipment and facilities, including:

growth cabinets

culture vessels, growth chambers, sterile containers, culture plates, flasks and bottles

autoclaves

positive filtration apparatus

auto pipettes and pipette pumps

cell counting chambers and a haemocytometer

incubators, including specialised atmosphere carbon dioxide

binocular light inverted microscopes

centrifuges

workplace procedures and standard methods.

Assessors must satisfy the NVR/AQTF mandatory competency requirements for assessors.

# Links

MSL Laboratory Operations Companion Volume Implementation Guide is available from VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=5c63a03b-4a6b-4ae5-9560-1e3c5f462baa