# MSL974033 Perform food tests

Release: 1

MSL974033 Perform food tests

# Modification History

Release 1. Supersedes and equivalent to MSL974020 Perform food tests. Element 7 integrated into Element 5. Minor changes to Performance Criteria. Microbiology and molecular tests added as options to Performance Evidence. Changes to Knowledge Evidence to remove or merge duplicating items.

# Application

This unit of competency describes the skills and knowledge to interpret food test requirements, prepare samples, conduct pre-use and calibration checks on equipment, and perform routine testing of raw food materials, in-process materials and final products. These tests will involve several measurement steps. The unit includes data processing and some interpretation of results and tracking of obvious test malfunctions where the procedure is standardised. However, personnel are not required to analyse data, optimise tests/procedures for specific samples or troubleshoot equipment problems where the solution is not apparent.

This unit applies to laboratory or technical assistants and instrument operators working in the food and beverage processing industry sectors.

No licensing or certification requirements exist at the time of publication. However, regulations and/or external accreditation requirements for laboratory operations exist, so local requirements should be checked. Relevant legislation, industry standards and codes of practice within Australia must also be applied.

# Pre-requisite Unit

Nil.

# Competency Field

Testing

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Interpret and schedule test requirements | 1.1 Review test request to identify samples to be tested, test method and equipment/instruments involved 1.2 Identify hazards and workplace controls associated with the sample, preparation/test methods, reagents and/or equipment 1.3 Plan parallel work sequences to optimise throughput of multiple sets of samples |
| 2. Receive and prepare food samples | 2.1 Register samples in accordance with workplace procedures, recording details that allow accurate tracking and chain of custody traceable to employee 2.2 Match sample provided to accompanying documentation and report any discrepancies in accordance with workplace procedures 2.3 Prepare samples and standards in accordance with food testing requirements |
| 3. Check equipment before use | 3.1 Set up equipment/instruments in accordance with test method requirements 3.2 Perform pre-use and safety checks in accordance with relevant workplace and operating procedures 3.3 Check equipment calibration using specified standards and procedures 3.4 Inspect reagents required for quality issues including visual checks and expiry |
| 4. Test samples to determine food components and characteristics | 4.1 Operate equipment/instruments in accordance with test method requirements 4.2 Perform tests and/or procedures on samples and standards in accordance with specified methods 4.3 Shut down equipment in accordance with operating procedures |
| 5. Process, analyse and record data | 5.1 Record test data noting atypical observations 5.2 Interpret graphs and compute results for all samples from these graphs 5.3 Ensure calculated values are consistent with reference standards and expectations 5.4 Record and report results in accordance with workplace procedures 5.5 Determine if basic procedure or equipment problems have led to atypical data or results 5.6 Follow procedures to ensure traceability of sample from receipt to reporting of results |
| 6. Maintain a safe work environment | 6.1 Use established safe work practices and personal protective equipment (PPE) to ensure personal safety and that of other laboratory personnel 6.2 Minimise the generation of wastes and environmental impacts 6.3 Ensure the safe collection of laboratory and hazardous waste for subsequent disposal 6.4 Care for and store equipment and reagents in accordance with workplace procedures |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential to performance.

Numeracy skills to:

process data, interpret gross features of data/results and make relevant conclusions

Problem-solving skills to:

identify atypical results as either out-of-normal range or an artefact

trace and source obvious causes of an artefact

track obvious test malfunctions for standardised procedures and troubleshoot basic equipment and methods.

Other foundation skills essential to performance are explicit in the performance criteria of this unit.

# Unit Mapping Information

Supersedes and is equivalent to MSL974020 Perform food tests.

# Links

MSL Laboratory Operations Companion Volume Implementation Guide is available from VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=5c63a03b-4a6b-4ae5-9560-1e3c5f462baa

Assessment Requirements for MSL974033 Perform food tests

# Modification History

Release 1. Supersedes and equivalent to MSL974020 Perform food tests. Element 7 integrated into Element 5. Minor changes to Performance Criteria. Microbiology and molecular tests added as options to Performance Evidence. Changes to Knowledge Evidence to remove or merge duplicating items.

# Performance Evidence

There must be evidence the candidate has completed the tasks outlined in the elements and performance criteria of this unit, and:

performed at least 5 routine food tests or procedures involving several measurement steps, including at least 4 of the following:

sensory tests

visual inspection

package integrity and compatibility test

physical/mechanical tests

chemical tests

thermal tests including stability of products and effectiveness of heat treatments

prepared samples using 3 different processes including at least 2 of the following:

grinding and milling

preparation of discs

digestion

extracting

refluxing

degassing

enzymatic treatment

washing and drying

ashing and temperature equilibration

precipitation and centrifugation

filtration

flocculation

evaporation.

# Knowledge Evidence

There must be evidence the candidate has knowledge of:

basic principles and concepts underpinning the food tests or procedures including:

structure and properties of proteins, lipids, carbohydrates, vitamins and minerals

food additives, flavourings and essences

nutrient value of major food groups

interaction of water with food components

chemical and microbial changes in food

food preservation techniques including atmosphere and modified atmosphere in relation

basic biochemistry of fermentation reactions

function of packaging for product integrity and controlled atmosphere

quality control program for raw materials, process control and finished product inspection

principles and purpose of food tests performed as part of the job role

basic principles and concepts related to equipment and instrument operation

function of key components of equipment/instrument and effects on the test of modifying equipment and instrument variables

concepts of metrology, including:

all measurements are estimates

precision, accuracy and significant figures

sources of error, uncertainty and repeatability

traceability

common sample preparation processes relevant to job role

types of food tests and procedures:

sensory tests

visual tests of content and package

physical/mechanical tests

chemical analysis

microbiological tests

molecular tests

thermal tests including stability of products and effectiveness of heat treatments

workplace and/or legal traceability requirements

awareness of environmental sustainability issues as they relate to the work task

legal, ethical and work health and safety (WHS) requirements specific to the work task.

# Assessment Conditions

Skills must have been demonstrated in the workplace or in a simulated environment that reflects workplace conditions and contingencies. The following conditions must be met for this unit:

use of suitable facilities, equipment and resources, including:

a standard laboratory equipped with test equipment, instruments, standards and materials

workplace procedures and standard methods

records, including:

test and calibration results

equipment use, maintenance and servicing history.

Assessors must satisfy the NVR/AQTF mandatory competency requirements for assessors

# Links

MSL Laboratory Operations Companion Volume Implementation Guide is available from VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=5c63a03b-4a6b-4ae5-9560-1e3c5f462baa