# MSL975027 Classify building sites

Release: 1

MSL975027 Classify building sites

# Modification History

Release 1. Supersedes and is equivalent to MSL976001A Classify building sites

# Application

This unit of competency covers the ability to classify building sites, including residential, light industrial, commercial and institutional structures for the purpose of providing guidance for the design of footing systems.

This unit of competency is applicable to senior technical officers working in the construction materials testing industry sector. Site classification is applicable to single dwelling houses, townhouses and commercial, institutional or light industrial buildings. The classification depends on reactivity of the foundation soils and other potential problems, such as mine subsidence, groundwater conditions and slope. These influence the design of footings, so as to minimise damage due to foundation movement during the life of the building. Operations are performed in accordance with laboratory and/or workplace procedures, and appropriate legislative requirements.

This competency is typically performed by paraprofessionals working under the supervision of an engineer.

While no specific licensing or certification requirements apply to this unit at the time of publication, laboratory operations are governed by relevant legislation, regulations and/or external accreditation requirements. Local requirements should be checked.

# Pre-requisite Unit

| MSL974014 | Classify soils |
| --- | --- |
| MSL973010 | Conduct laboratory-based acceptance tests for construction materials |
| MSL973009 | Conduct field-based acceptance tests for construction materials |

# Competency Field

Testing

# Unit Sector

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe the performance needed to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Prepare to classify site | 1.1 | Review job request and any accompanying documentation to identify purpose of site classification, proposed structure, client’s information needs and timeframe |
|  |  | 1.2 | Review relevant building codes or regulations, standards and workplace procedures for classifying site |
|  |  | 1.3 | Locate any previous site classifications conducted in same area |
|  |  | 1.4 | Review supplied site drawings, topography, borehole locations and logs, photographs and design plans for planned structure |
|  |  | 1.5 | Inspect supplied samples and investigation data for completeness and obvious errors |
|  |  | 1.6 | Confirm that investigation data meets depth requirements for the type of site and location |
|  |  | 1.7 | Liaise with client when supplied documentation, samples, investigation data and/or job request do not comply with workplace procedures and resolve any problems |
| 2 | Analyse site information and test results | 2.1 | Locate and assess site features, such as slope, structures, drainage, trees and vegetation or abnormal soil conditions that may affect site classification |
|  |  | 2.2 | Determine if fill is present on-site, whether controlled or uncontrolled, and its depth and extent |
|  |  | 2.3 | Determine bearing capacity of soil |
|  |  | 2.4 | Review shrink/swell or correlated clay index test results to determine soil reactivity |
|  |  | 2.5 | Review bore log data in conjunction with soil reactivity and calculate potential soil movement |
|  |  | 2.6 | Recognise obvious errors, insufficient or inconsistent data and limitations of test results |
|  |  | 2.7 | Arrange for, or conduct, additional sampling and/or testing to ensure reliable data for site classification, as required |
| 3 | Report site classification | 3.1 | Use standard symbols and descriptors to assign a valid site classification that accurately reflects soil reactivity, surface movement and any problematic features |
|  |  | 3.2 | Summarise field observations, calculations and test results used to determine site classification |
|  |  | 3.3 | Accurately describe site features that will affect the design of the proposed structure |
|  |  | 3.4 | Provide adequate data to inform footing system design |
|  |  | 3.5 | Authorise release of site classification report to the client, or forward the report to a signatory for review and release |
| 4 | Finalise site classification | 4.1 | Store client’s documentation, samples and test data, and site classification in accordance with workplace procedures |
|  |  | 4.2 | Maintain confidentiality of client information |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential to performance.

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Range of Conditions

| This field allows for different work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) are included. |  |
| --- | --- |
| Standards, codes, procedures and/or workplace requirements | Standards, codes, procedures and/or workplace requirements include the latest version of one or more of: Australian and international standards covering the requirements for the competence of testing and calibration laboratories, laboratory safety, and quality and environmental management national work health and safety (WHS) standards and codes of practice, national measurement regulations and guidelines, and environmental legislation and regulations specific codes, guidelines, standards and methods, such as: State/Territory Building Codes and Regulations AS 2870-2011 Residential slabs and footings AS 3798-2007 Guidelines on earthworks for commercial and residential developments AS 1726-1993 Geotechnical site investigations AS 1289 Set Methods of testing soils for engineering purposes National Association of Testing Authorities (NATA) documents regarding construction materials testing (Field application document) workplace documents, such as standard operating procedures (SOPs); quality and equipment manuals; calibration and maintenance schedules; material safety data sheets (MSDS) and safety procedures; material, production and product specifications; production and laboratory schedules; workplace recording and reporting procedures; waste minimisation and safe disposal procedures; maps and site plans sampling and testing procedures for specific sites, clients and samples |
| WHS and environmental management requirements | WHS and environmental management requirements include:  complying with WHS and environmental management requirements at all times, which may be imposed through state/territory or federal legislation. These requirements must not be compromised at any time applying standard precautions relating to the potentially hazardous nature of samples |

# Unit Mapping Information

Release 1. Supersedes and is equivalent to MSL976001A Classify building sites

# Links

MSA Training Package Implementation Guides - http://mskills.org.au/training-packages/info/