# Assessment Requirements for MSMPMC300 Produce structural precast concrete

Release: 1

Assessment Requirements for MSMPMC300 Produce structural precast concrete

# Modification History

Release 1. New unit. Supersedes and is equivalent to PMC553051 Produce structural precast concrete

# Performance Evidence

There must be evidence the candidate has completed the tasks outlined in the elements and performance criteria of this unit, and:

produced at least 1 structural precast concrete product, including:

monitoring the following key variables to ensure product integrity and conformance to specification:

mould condition

amount and consistency of concrete mix

placement and anchorage of reinforcement and fittings

placement of voids and ducts

tension of reinforcement

concrete cover

compaction and vibration

curing conditions

curing time and rate

stripping strength

recognising possible causes of the following routine problems:

raw material and mix variations

mould anomalies

casting and operating conditions

reinforcement abnormalities

process errors.

# Knowledge Evidence

There must be evidence the candidate has knowledge of:

principles of casting process

significance to structural concrete products of key factors, including:

concrete cover

role of reinforcement

placement and anchorage of reinforcement and fittings

stressing and destressing

curing

compaction and vibration

formwork and specialised formwork (including voids and ducts)

mould design

mould condition and wear

water and cement ratio

concrete finishing

methods of:

curing

compaction

fabrication

concrete sampling and testing procedures

the potential effects of variations in raw materials and equipment operation and processes in relation to quality of product

quality requirements at each stage of production

common adjustments in variables and their impact on product quality and production output

routine and non-routine problems, their probable causes and relevant corrective actions

hazards from critical stages of pre-stressing reinforcement (placement, stressing, de-stressing, cutting)

work health and safety (WHS) requirements relevant to work outcome

structural precast and pre-stressed concrete production hazards and:

their possible causes

potential consequences

risk control procedures.

# Assessment Conditions

Skills must have been demonstrated in the workplace or in a simulated environment that reflects workplace conditions and contingencies. The following conditions must be met for this unit:

use of suitable facilities, equipment and resources, including:

structural precast concrete equipment and ancillary equipment

materials to be processed

specifications, procedures and job sheets

modelling of industry operating conditions, including:

routine and non-routine problems.

Assessors must satisfy the NVR/AQTF mandatory competency requirements for assessors.

# Links

Companion Volume Implementation Guides are available at VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=d1287d36-dff4-4e9f-ad2c-9d6270054027