# MSMPMC321 Batch mix concrete

Release: 1

MSMPMC321 Batch mix concrete

# Modification History

Release 1. New unit. Supersedes and is equivalent to PMC552060 Batch mix concrete

# Application

This unit describes the skills and knowledge required to batch mix concrete. It applies to concrete batching plants in the pre-mixed and manufactured concrete products sectors. It applies to wet and dry batching.

This unit applies to operators who are required to take responsibility for own outputs and follow established procedures to select, batch and load materials; operate and adjust batching and/or mixing equipment and discharge batched concrete. Operators are also required to work autonomously and apply their own judgement to interpret information; monitor processes; identify and address routine and non-routine problems and control hazards.

This unit applies to an individual working alone or as part of a team or group and in liaison with other shift team members, team leader and supervisor.

No licensing or certification requirements exist at the time of publication. Relevant legislation, industry standards and codes of practice within Australia must be applied.

# Pre-requisite Unit

Nil

# Competency Field

Operations

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Determine mix requirements | 1.1 Identify job requirements, standards and specifications 1.2 Select the correct type of materials 1.3 Calculate quantity of materials required 1.4 Identify any material handling problems and take the appropriate action 1.5 Update material records |
| 2. Batch mix raw materials | 2.1 Set up and operate batching and/or mixing equipment to specifications and procedures 2.2 Batch materials as required by specifications and procedures 2.3 Check that materials prepared match requirements 2.4 Use ancillary equipment as required according to procedures 2.5 Discharge batched concrete into delivery trucks or mixers |
| 3. Monitor batch mixing process | 3.1 Check and adjust settings 3.2 Make routine checks and recognise developing problems 3.3 Identify equipment in need of maintenance or repair 3.4 Take samples and interpret test results 3.5 Check the production process and product complies with the appropriate quality procedures 3.6 Complete all required records |
| 4. Maintain batch mixing plant and area | 4.1 Keep the area and equipment clean and in good order 4.2 Shut down equipment 4.3 Complete maintenance checklists |
| 5. Rectify routine problems | 5.1 Identify and rectify faults and equipment failure 5.2 Maintain records and log books of equipment operations 5.3 Identify non-routine problems and report to designated person |
| 6. Control hazards | 6.1 Identify batching hazards 6.2 Assess the risks arising from those hazards 6.3 Implement measures to control those risks |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential to performance.

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Unit Mapping Information

Release 1. New unit. Supersedes and is equivalent to PMC552060 Batch mix concrete

# Links

Companion Volume Implementation Guides are available at VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=d1287d36-dff4-4e9f-ad2c-9d6270054027