# MSMRV357 Remove and replace external components of a recreational vehicle

Release: 1

MSMRV357 Remove and replace external components of a recreational vehicle

# Modification History

Release 1 - New unit.

# Application

This unit of competency covers the skills and knowledge required to remove and replace the roof hatches, windows and external doors on a recreational vehicle. The same hatches, windows and doors may be replaced, or the removed hatches, windows and doors may be replaced with different hatches, windows and doors. This unit also covers the resealing of hatches, windows and doors. The work will be carried out in a recreational vehicle service and repair facility.

This unit of competency covers the preparation for, removal of, replacement of, and clean-up after the removal, replacement and resealing of hatches, windows and doors.

The operator will be required to work to procedures, and follow plans/specifications, insurance requirements and customer needs.

This unit of competency applies to an individual working alone or as part of a team/work group and working in liaison with other shift team members.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil

# Competency Field

Operations

# Unit Sector

# Elements and Performance Criteria

| Elements describe the essential outcomes |  | Performance criteria describe the performance needed to demonstrate achievement of the element |  |
| --- | --- | --- | --- |
| 1 | Plan the work | 1.1 | Review the requirements of the work, including insurance requirements and customer needs |
|  |  | 1.2 | Examine hatches, windows and doors, determine what needs to be done and how to do it |
|  |  | 1.3 | Check required tools and jigs are available at the worksite |
|  |  | 1.4 | Check required materials and components are available at the worksite |
|  |  | 1.5 | Identify any hazards and the related hazard controls |
|  |  | 1.6 | Check that hazard controls are in place and operational |
|  |  | 1.7 | Query any items that are different to usual and clarify requirements |

| 2 | Remove hatches, windows and doors in accordance with devised procedures | 2.1 | Use relevant hazard controls, including personal protective equipment (PPE) |
| --- | --- | --- | --- |
|  |  | 2.2 | Remove hatches, windows and doors maximising the potential for reuse |
|  |  | 2.3 | Segregate and handle waste in accordance with procedures |
|  |  | 2.4 | Temporarily store hatches, windows and doors which are to be reused |

| 3 | Refurbish components as required | 3.1 | Use relevant hazard controls, including PPE |
| --- | --- | --- | --- |
|  |  | 3.2 | Examine all removed items and the adjacent parts of the caravan which have been revealed |
|  |  | 3.3 | Identify any additional damage revealed and decide how it can be made good |
|  |  | 3.4 | Negotiate with customer over any changes in job scope and cost |
|  |  | 3.5 | Make good/replace components as required |

| 4 | Replace hatches, windows and doors in accordance with devised procedures | 4.1 | Use relevant hazard controls, including PPE |
| --- | --- | --- | --- |
|  |  | 4.2 | Replace hatches, windows and doors in accordance with the devised procedure |
|  |  | 4.3 | Ensure items are attached as required |
|  |  | 4.4 | Adjust as required |

| 5 | Reseal hatches, windows and doors in accordance with devised procedures | 5.1 | Use relevant hazard controls, including PPE |
| --- | --- | --- | --- |
|  |  | 5.2 | Replace seals/sealing material in accordance with the devised procedure |
|  |  | 5.3 | Adjust/trim as required |
|  |  | 5.4 | Ensure seals are effective |

| 6 | Finish removal and replacement of hatches, windows and doors | 6.1 | Check hatches, windows and doors have been refitted correctly and make any required adjustments |
| --- | --- | --- | --- |
|  |  | 6.2 | Clean up recreational vehicle and make ready for hand back to customer |
|  |  | 6.3 | Clean up work area, leaving it ready for the next job |
|  |  | 6.4 | Segregate and handle waste in accordance with procedures |
|  |  | 6.5 | Complete any required documentation/records |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential to performance.

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Range of Conditions

| This field allows for different work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) are included. |  |
| --- | --- |
| Regulatory framework | The latest version of all legislation, regulations, industry codes of practice and Australian/international standards, or the version specified by the local regulatory authority, must be used, and include one or more of the following: legislative requirements, including work health and safety (WHS) industry codes of practice and guidelines environmental regulations and guidelines Australian and other standards licence and certification requirements |
| Procedures | All operations must be performed in accordance with relevant procedures. Procedures are written, verbal, visual, computer-based or in some other form, and include one or more of the following: work instructions standard operating procedures (SOPs) safe work method statements (SWMS) temporary instructions any similar instructions provided for the smooth running of the plant |
| Hazards | Hazards include one or more of the following: heat, dust or other atmospheric hazards electricity gas equipment failures industrial (machinery, equipment and product) noise, rotational equipment or vibration hazardous products and materials unauthorised personnel sharp edges, protrusions or obstructions slippery surfaces, spills or leaks other hazards that might arise |
| Routine problems | Routine problems must be resolved by applying known solutions. Routine problems are predictable and include one or more of the following: equipment/plant/machine malfunction variations in process conditions variations in materials or contamination of materials equipment, tool, die or mould damage product faults tooling problems Known solutions are drawn from one or more of the following: procedures training remembered experience Non-routine problems must be reported according to relevant procedures. |

# Unit Mapping Information

No equivalent unit

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=d1287d36-dff4-4e9f-ad2c-9d6270054027