# MSMRV358 Modify the body of a recreational vehicle

Release: 1

MSMRV358 Modify the body of a recreational vehicle

# Modification History

Release 1 - New unit.

# Application

This unit of competency covers the skills and knowledge required to make body modifications, such as installing additional access doors, drawers and similar modifications, to a recreational vehicle. The work will be carried out in a recreational vehicle service and repair facility.

This unit of competency covers the preparation for, modification of and clean-up after the modification.

The operator will be required to work to procedures, and follow plans/specifications, insurance requirements and customer needs.

This unit of competency applies to an individual working alone or as part of a team/work group and working in liaison with other shift team members.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil

# Competency Field

Operations

# Unit Sector

# Elements and Performance Criteria

| Elements describe the essential outcomes |  | Performance criteria describe the performance needed to demonstrate achievement of the element |  |
| --- | --- | --- | --- |
| 1 | Plan the work | 1.1 | Review the requirements of the work, including insurance requirements and customer needs |
|  |  | 1.2 | Examine recreational vehicle and identify likely problems with the proposed location or nature of the modification |
|  |  | 1.3 | Negotiate exact details of proposed modification with customer |
|  |  | 1.4 | Confirm what needs to be done and determine how to do it |
|  |  | 1.5 | Check required tools and jigs are available at the worksite |
|  |  | 1.6 | Check required materials and components are available at the worksite |
|  |  | 1.7 | Identify any hazards and the related hazard controls |
|  |  | 1.8 | Check that hazard controls are in place and operational |
|  |  | 1.9 | Query any items that are different to usual and clarify requirements |

| 2 | Make the required penetration in accordance with devised procedures | 2.1 | Use relevant hazard controls, including personal protective equipment (PPE) |
| --- | --- | --- | --- |
|  |  | 2.2 | Remove recreational vehicle components as required maximising the potential for reuse |
|  |  | 2.3 | Cut through external cladding, interior panelling and frame as required |
|  |  | 2.4 | Segregate and handle waste in accordance with procedures |
|  |  | 2.5 | Temporarily store removed components which are to be reused |

| 3 | Refurbish as required | 3.1 | Use relevant hazard controls, including PPE |
| --- | --- | --- | --- |
|  |  | 3.2 | Examine all removed items and the adjacent parts of the caravan which have been revealed |
|  |  | 3.3 | Identify any damage revealed and decide how it can be made good |
|  |  | 3.4 | Negotiate with customer over any changes in job scope and cost |
|  |  | 3.5 | Make good/replace as required |

| 4 | Install the required items in accordance with devised procedures | 4.1 | Use relevant hazard controls, including PPE |
| --- | --- | --- | --- |
|  |  | 4.2 | Install required items in accordance with the devised procedure |
|  |  | 4.3 | Ensure items are attached as required |
|  |  | 4.4 | Adjust as required |
|  |  | 4.5 | Ensure items function as intended |

| 5 | Seal installed items in accordance with devised procedures | 5.1 | Use relevant hazard controls, including PPE |
| --- | --- | --- | --- |
|  |  | 5.2 | Apply seals/sealing material in accordance with the devised procedure |
|  |  | 5.3 | Adjust/trim as required |
|  |  | 5.4 | Ensure seals are effective |

| 6 | Finish body modifications | 6.1 | Check all installed items have been fitted correctly and make any required adjustments |
| --- | --- | --- | --- |
|  |  | 6.2 | Clean up recreational vehicle and make ready for hand back to customer |
|  |  | 6.3 | Clean up work area, leaving it ready for the next job |
|  |  | 6.4 | Segregate and handle waste in accordance with procedures |
|  |  | 6.5 | Complete any required documentation/records |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential to performance.

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Range of Conditions

| This field allows for different work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) are included. |  |
| --- | --- |
| Regulatory framework | The latest version of all legislation, regulations, industry codes of practice and Australian/international standards, or the version specified by the local regulatory authority, must be used, and include one or more of the following: legislative requirements, including work health and safety (WHS) industry codes of practice and guidelines environmental regulations and guidelines Australian and other standards licence and certification requirements |
| Procedures | All operations must be performed in accordance with relevant procedures. Procedures are written, verbal, visual, computer-based or in some other form, and include one or more of the following: work instructions standard operating procedures (SOPs) safe work method statements (SWMS) temporary instructions any similar instructions provided for the smooth running of the plant |
| Hazards | Hazards include one or more of the following: heat, dust or other atmospheric hazards electricity gas equipment failures industrial (machinery, equipment and product) noise, rotational equipment or vibration hazardous products and materials unauthorised personnel sharp edges, protrusions or obstructions slippery surfaces, spills or leaks other hazards that might arise |
| Routine problems | Routine problems must be resolved by applying known solutions. Routine problems are predictable and include one or more of the following: equipment/plant/machine malfunction variations in process conditions variations in materials or contamination of materials equipment, tool, die or mould damage product faults tooling problems Known solutions are drawn from one or more of the following: procedures training remembered experience Non-routine problems must be reported according to relevant procedures. |

# Unit Mapping Information

No equivalent unit

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=d1287d36-dff4-4e9f-ad2c-9d6270054027

Assessment Requirements for MSMRV358 Modify the body of a recreational vehicle

# Modification History

Release 1 - New unit.

# Performance Evidence

Evidence required to demonstrate competence in this unit must be relevant to and satisfy the requirements of the elements and performance criteria and demonstrate the ability to:

complete body modifications to at least three (3) recreational vehicles

interpret requirements and plan work

identify hazards and apply relevant hazard controls

recognise situations requiring action and take the appropriate action

communicate effectively with customers, team/work group and supervisors.

# Knowledge Evidence

Evidence must be provided that demonstrates knowledge of:

hazards that may arise in the job/work environment

modification procedures and the reasons for those procedures

materials and tools to be used and their correct usage

checks and adjustments to be made.

# Assessment Conditions

The unit should be assessed holistically and the judgement of competence based on a holistic assessment of the evidence.

The collection of performance evidence:

should occur over a range of situations which include typical disruptions to normal, smooth operation

will typically include a supervisor/third-party report or other evidence, focusing on consistent performance and problem recognition and solving. A supervisor/third-party report must be prepared by someone who has a direct, relevant, current relationship with the person being assessed and who is in a position to form a judgement on workplace performance relevant to the unit of competency

may use industry-based simulation for all or part of the unit particularly where safety, lack of opportunity or significant cost is an issue.

Assessment should occur in operational workplace situations. Where this is not possible, or where personal safety or environmental damage are limiting factors, assessment must occur in a sufficiently rigorous simulated environment reflecting realistic operational workplace conditions. This must cover all aspects of workplace performance, including environment, task skills, task management skills, contingency management skills and job role environment skills.

Knowledge evidence may be collected concurrently with performance evidence (provided a record is kept) or through an independent process, such as workbooks, written assessments or interviews (provided a record is kept).

Assessment processes and techniques must be appropriate to the language, literacy and numeracy requirements of the work being performed and the needs of the candidate.

Conditions for assessment must include access to all tools, equipment, materials and documentation required, including relevant workplace procedures, product and manufacturing specifications associated with this unit.

The regulatory framework will be reflected in workplace policies and procedures and is not required to be independently assessed.

Foundation skills are integral to competent performance of the unit and should not be assessed separately.

As a minimum, assessors must satisfy the Standards for Registered Training Organisations 2015 assessor requirements.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=d1287d36-dff4-4e9f-ad2c-9d6270054027