# Assessment Requirements for MSTDC3005 Perform spotting function in dry cleaning operations

Release: 1

Assessment Requirements for MSTDC3005 Perform spotting function in dry cleaning operations

# Modification History

Release 1. Supersedes and is equivalent to MSTDC3001 Perform spotting function in dry cleaning operations.

# Performance Evidence

There must be evidence the candidate has completed the tasks outlined in the elements and performance criteria of this unit, and:

performed spotting techniques on at least 10 different articles received for dry cleaning, covering at least 5 different fabrics, including:

determining spotting requirements and suitable spotting agents for different stains

carrying out spotting procedures

recorded non-removable stains in accordance with workplace procedures.

# Knowledge Evidence

There must be evidence the candidate has knowledge of:

safe work practices and procedures and use of personal protective equipment (PPE)

key requirements of Australian Standards, industry regulations and codes of practice that govern the control of chemicals in the workplace, including:

Managing risks of hazardous chemicals in the workplace: Model Code of Practice, Safe Work Australia

Managing risks of exposure to solvents in the workplace, Information sheet, Safe Work Australia

Dry Cleaning Institute of Australia (DIA) solvent information resources

the range of spotting agents, how they are classified and their application to specific stains

spotting resource requirements:

blotters

cheesecloth

drying cabinet

spatula

spotting board

spotting brushes

towel

tumbler

application procedures for spotting agents

the differences between oxidising and reducing bleaches

rules for bleaching

acceleration processes for spotting agents

safety and handling and storage requirements of spotting agents

stain identification procedures, including steps to be followed, to recognise typical stains, including:

albumin stains

dye stains

organic or non-organic

protein stains

the characteristics and qualities of special fabrics

requirements for handling and care of fabrics during the spotting process, including:

brushing requirements

procedures for use of certain equipment and chemicals

rinsing needs

side of fabric upon which to apply spotting

special needs for keeping fabric flat or not

susceptibility to breaks and creasing

tamping requirements

characteristics of fibres, fabrics, and dyes or colours, and the effects of the spotting procedure

factors affecting the spotting process:

moisture

pH concentration

temperature

time

special spotting agents, including wet-side and dry-side agents:

acetone

detergents

digestion agents

formic acid

iodine

methyl alcohol

oxalic acid

oxidising and reducing bleaches

sodium thiosulphate

solvents

advanced spotting processes and procedures, including:

ammonia action

bleaching

chemical action

digestion

oxidising

reducing

steam treatment

synthetic detergent treatment

regulatory requirements for disposing of chemicals and solvents.

# Assessment Conditions

Skills must have been demonstrated in the workplace or in a simulated environment that reflects workplace conditions and contingencies. The following conditions must be met for this unit:

use of suitable facilities, equipment and resources, including:

a dry cleaning environment set up with equipment and agents for spot cleaning stains and related standard operating procedures (SOPs)

articles made of different fabrics, with different stains, to be spot cleaned.

Assessors must satisfy the NVR/AQTF mandatory competency requirements for assessors.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=a203ec5c-de7d-406b-b3e1-8f1a9b76e92e