# Assessment Requirements for MSTFR2004 Replace half leather sole

Release: 1

Assessment Requirements for MSTFR2004 Replace half leather sole

# Modification History

Release 1. Supersedes and is equivalent to LMTFR2004A Replace half leather sole

# Performance Evidence

Evidence required to demonstrate competence in this unit must be relevant to and satisfy the requirements of the elements and performance criteria, and include:

reading and following work instructions, standard operating procedures (SOPs), safe work practices

applying relevant standards

inspecting shoes, identifying repair materials and advising customer on costs prior to commencing shoe repair

determining appropriate method of replacing sole and heel, and selecting, checking and preparing appropriate tools and materials

removing top lifts and soles back to waist, ensuring no damage to the shoe, and selecting replacement sole material of suitable thickness, and top pieces and lifts, appropriate to original footwear

marking waist using the new half sole as a guide, ensuring waist marks are equal on both items of footwear

skiving half sole in a half moon shape, and tapering from the waist mark to the toe consistent with workplace requirements

securing lasting margin, patching where necessary, and checking or replacing fillers as required

scouring half sole prior to securing

applying hand and machine techniques over the range of required operations

using appropriate adhesive and primer according to manufacturer and workplace requirements

repairing half leather soles on at least three (3) pairs of shoes to meet quality and enterprise standards

checking quality of adhesion, adjusting, as required, and stitching sole using a sole stitching machine, if required

trimming, inking or staining edges and heel lifts to match original shoe colour, polishing and finishing off

packaging, labelling and storing repaired shoes and maintaining records.

# Knowledge Evidence

Evidence must be provided that demonstrates knowledge of:

safe work practices and procedures and use of personal protective equipment (PPE)

relevant Australian Standards

full range of footwear styles and their structure or construction

characteristics and use of adhesives and primers and their safe application and disposal

characteristics of typical leathers, fabrics, and other materials used in shoe construction and repair

use of machines, equipment and tools, adhesives and primers, stains, dyes and inks

tools:

shoe hammer

tack hammer

shoe knife

sharpening stick

peg awl

glue gun

pincers

nippers

lasting pliers

shears

drag knife

tack

machines:

finishing machine

heat activator

last

heat pliers

quality standards and leather handling procedures

repair and finish of shoes particularly in relation to use of machines, equipment and tools, adhesives and primers, stains, dyes and inks

work health and safety (WHS) and environmental aspects of relevant repair processes

general industry housekeeping policies and procedures

quality practices

workplace procedures

practices for recording and reporting.

# Assessment Conditions

Assessors must:

satisfy the assessor competency requirements that are in place at the time of the assessment, as set by the VET regulator

have vocational competency in replacing half leather sole, at least to the level being assessed, with relevant industry knowledge and experience.

Assessment should occur in operational workplace situations. Where this is not possible, or where personal safety or environmental damage are limiting factors, assessment must occur in a sufficiently rigorous simulated environment reflecting realistic operational workplace conditions. This must cover all aspects of workplace performance, including environment, task skills, task management skills, contingency management skills and job role environment skills.

Assessment processes and techniques must be appropriate to the language, literacy and numeracy requirements of the work being performed and the needs of the candidate.

Conditions for assessment must include access to all tools, equipment, materials and documentation required, including relevant workplace procedures, product and manufacturing specifications associated with this unit.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=a203ec5c-de7d-406b-b3e1-8f1a9b76e92e